Day in the Life of the Hudson River 10/14/21

RANDALL’S &WARD’S ISLAND, MANHATTAN

RIVER MILE: Harlem River 1

Salt Front: ~ RM 49

Dwight School- Emma Ryan and 4 teachers

60 students, 7th grade

Randall’s Island- Chris Girgenti and staff

Latitude 40.786015 N & Longitude – 73.934763 W

Location:  Randall’s Island Park, Harlem River, Manhattan, NY

Area: Randall’s Island Park, Harlem River Waterfront (North of 103rd St. bridge/Ward’s Island Bridge), sandy adjacent to shore, increasingly muddy with depth; sporadic rocks

Surrounding Land Use: 50% Urban/Residential, 40% Parkland, 10% Beach

Sampling Site: Grassy, beach, banks altered, wood bulkhead, concrete bulkhead, riprap, pipe entering water

Shoreline: 90% Sandy; 10% Rocky

Water:  calm

River Bottom: 60% Rocky, 40% Sandy

Plants in water: no plants in water area

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature |  | 9:48 AM 10:31 AM 11:39 AM 12:12 PM |  | 19 o C 21 o C 20 o C 23 o C |  |  | 66.2 o F 69.8 o F 68 o F 73.4 o F |  |  |
| Wind Speed: |  | 10:36 AM 12:03 PM 12:12 PM |  | 10 mph 6.2 mph 6.4 mph |  |  |  |  |  |
| Weather today |  | 9:49 AM- 10:29 AM 12:30 PM |  | 31% cloud cover 34% cloudy cover 25% cloud cover |  |  | No Rain No Rain No Rain |  |  |
| Weather the past 3 days: |  | Partly cloudy without rain the previous 3 days |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Height (cm) |  |  |
|  |  | NA |  |  |  |  |  |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |  |
|  |  | 10:48 AM 11:53 AM 12:30 PM |  | 4 12.7 36.6 |  | 0.078 0.247 0.713 | N/R | N/R |  |
| Water Temperature- |  | 9:55 AM 10:33 AM 11:41 AM 12:18 PM |  | 19.5 o C 19.8 o C 20.3 o C 20.0 o C |  |  |  |  |  |
| Turbidity- Long sight tube |  | 10:05 AM 10:37 AM 12:19 PM |  | 26 cm 31.3 cm 43.6 cm |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity- Hydrometer |  | 10:00 AM 10:38 AM 11:46 AM 12:22 PM |  | 21.5 ppt 22.0 ppt 22.0 ppt 20.7 ppt |  |  |  |  |  |
| DO – |  | Time |  | Temp |  |  | ppm DO | % saturation |  |
| *DO, pH, Nitrates and Phosphates are all taken with a very low resolution wide range kit and not reflective of conditions in the water |  | 10:57 AM 12:05 PM 12:43 PM |  | ~20 o C ~20 o C ~20 o C |  |  | 2 ppm 2 ppm 2 ppm | 20% 20% 20% |  |
| *pH – |  | 10:08 AM 10:48 AM 12:03 PM 12:27 PM |  | 6 6 6.5 6 |  |  |  |  |  |
| *Nitrates- |  | 10:24 AM 11:00 AM 12:06 PM 12:37 PM |  | 5 ppm 5 ppm 5 ppm 20 ppm |  |  |  |  |  |
| *Phosphates- |  | 10:22 AM 12:05 PM 12:37 PM |  | 2 ppm 2 ppm 2 ppm |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |  |
| Fish |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
|  |  | 90 1 1 3 |  | Atlantic Silversides Striped Bass Blue Fish Porgy |  |  |  |  |  |
| Fish Totals |  | TOTAL: 95 |  | Diversity: 4 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
|  |  | 12 6 3 |  | Moon jellies Comb jellies Blue Crab |  |  |  |  |  |
| Macroinvertebrate Totals |  | TOTAL: 21 |  | Diversity: 3 |  |  |  |  |  |
| Sediment Core |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| (Group 1) NA (Group 2) NA (Group 3) 30 cm (Group 4) 30 cm | No Yes Yes Yes |  | Yes No Yes - |  | Yes Yes No No |  | Abundant: Mud, sand, gravel, pebble Rare: clay Abundant: mud, sand, pebbles Common: gravel Rare: clay Abundant: sand, gravel Common: clay, mud, pebbles Abundant: Sand Common: clay, mud, gravel, pebbles Rare: mud | Rare: shell oysters, freshwater mussels Rare: leaves, wood, oyster shells, brick, slag NA Rare: macroinvertebrates, brick |  |
| Boats |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial |  |  | Cruise ship NYPD Boat |  |  |  |  |  |  |
| Residential |  |  | Motorboat |  |  |  |  |  |  |