Day in the Life of the Hudson River 10/14/21

Pier 1 Lighthouse Museum, Staten Island

RIVER MILE: -4.5

Salt Front: ~ 49

Dave Scheffler & Curtis High School

12 students & 2 adults

Latitude 40.640951 N & Longitude – 74.073463 W

Location: Pier 1 Lighthouse Museum, Staten Island, NY

Area: Pier with parking lot.

Surrounding Land Use: 100% Urban

Sampling Site: Wood bulkhead, concreate bulkhead, banks altered, riprap

Shoreline: 20% sandy, 30% muddy, 50% rocky.

River Bottom: Rocky

Water : Calm

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature |  | 10:23 AM |  | 28.5 o C |  |  | 83.3 o F |  |  |
| Wind Speed: |  | 10:23 AM |  | Anemometer 2.6 mph |  |  | Beaufort NR | Direction SW |  |
| Weather today |  | 10:23 AM |  | <25% cloud cover |  |  | No rain |  |  |
| Weather the past 3 days: |  | Cloudy, humid, 70-80 degrees |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Height (cm) |  |  |
|  |  | 10:10 AM- 12:11 PM |  | Rising Rising |  |  | 213.36 cm 243.84 cm |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Slack |  |
|  |  | 10:39 AM 12:18 PM |  | 4.76 3.41 |  | 0.093 0.068 | South South | Ebb Ebb |  |
| Water Temperature- |  | 10:58 AM |  | 22o C |  |  | 72 o F |  |  |
| Turbidity- Long Sight Tube |  | 10:51 AM |  | 101.5 cm |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity- Hydrometer |  | 11:03 AM |  | 18.33 ppt |  |  |  |  |  |
| Standardized Salinity- NR |  |  |  |  |  |  |  |  |  |
| DO – NR |  | Time |  | Temp |  |  | ppm DO | % Saturation |  |
|  |  | NR |  | 22 o C |  |  | NR | NR |  |
| pH – Color Match Kit |  | 11:12 AM |  | 7.67 |  |  |  |  |  |
| Nitrates- strip tests – not a good tool for the field |  | 11:22 AM |  | Test results were extremely high 1 mg/L |  |  |  |  |  |
| Phosphates-strip tests - not a good tool for the field |  | 11:12 AM |  | Test results were extremely high 25mg/L |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |  |
| Fish - |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| *Oyster Cage |  |  |  |  |  |  |  |  |  |
| Fish Totals |  | TOTAL: 0 |  | Diversity: 0 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Macroinvertebrate Totals |  | TOTAL:0 |  | Diversity: 0 |  |  |  |  |  |
| Sediment Core |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| NR | No |  | Yes |  | NR |  | Abundant: clay Common: mud, sand | Common: macroinvertebrates, shells, living vegetation |  |
| Boats-NR |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial | 10:18 AM |  | Large Boats (unable to get name) |  | Dark Grey |  | Southbound | Loaded |  |
|  | 10:18 AM |  | Large Boats (unable to get name) |  | Red, Dark Blue & white |  | Southbound | NR |  |
|  | 11:22 AM |  | Large Boats (unable to get name) |  | White & Blue |  | Southbound | Light |  |