Day in the Life of the Hudson River 10/14/21

Henry Hudson Park, Bethlehem

RIVER MILE: 138

Salt Front: ~ 49

Sarah Loya & the 5th grade class of Albany Academy for Girls

18 students & 3 adults

Latitude 42.543455 N & Longitude – 73.759119 W

Location: Henry Hudson Park Waterfront, Bethlehem, Albany County, New York

Area: Park/boat launch, used for picnics, just north of Vloman Kill

Surrounding Land Use: 100% parkland with 75% vegetated coverage

Sampling Site: Grassy, parking lot, banks altered, riprap, bulkhead rocky

Plants in area: Water chestnuts – pulled out many of these

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature |  | 11:00 AM |  | 22 o C |  |  | 71.6 o F |  |  |
| Wind Speed: |  | 11:00 AM |  | Anemometer 1-3 mph |  |  | Beaufort 1 | Direction North |  |
| Weather today |  | 11:00 AM |  | 51-75% cloud cover |  |  | No rain |  |  |
| Weather the past 3 days: |  | Dry and warm. 50s in the AM and low 70s by PM |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Height (cm) |  |  |
|  |  | 10:00 AM-11:00 AM 1:00 PM |  | Start Rising Falling |  |  | Marked waters edge 8 cm change 6 cm change |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Slack |  |
|  |  | Full sampling period |  | NR |  | NR | South | Ebb |  |
| Water Temperature- |  | 11:00 AM |  | 20o C |  |  | 68 o F |  |  |
| Turbidity- Long Sight Tube |  | 10:00 AM 11:00 AM 12:45 PM |  | 71, 76 cm 85, 97 cm 77 cm |  |  | Average 73.5 cm Average 91 cm |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity- Low Range Quantabs |  | 11:00 AM |  | 1.0 to 1.2 quantab units (<32 ppm) |  |  | BD for this sampling instrument |  |  |
| Standardized Salinity- |  | NR |  | NR |  |  |  |  |  |
| DO – Hach Drop Count Kit |  | Time |  | Temp |  |  | ppm DO | % Saturation |  |
|  |  | 11:00 AM |  | 20 o C |  |  | 10 | NR |  |
| pH – Color Match Kit |  | 11:00 AM |  | 7.75 |  |  |  |  |  |
| Nitrates- |  |  |  | NR |  |  |  |  |  |
| Phosphates- |  |  |  | NR |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |  |
| Fish - |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| *9 seines, hard during high tide with rocks and concrete in the water |  |  |  |  |  |  |  |  |  |
| Fish Totals |  | TOTAL: 0 |  | Diversity: 0 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Macroinvertebrate Totals |  | TOTAL:0 |  | Diversity: 0 |  |  |  |  |  |
| Observations |  |  |  |  |  |  |  |  |  |
| AREA: Many water chestnut SEDIMENT CORE: smelled fishy and muddy, texture was smooth and gritty, colors were brown, grey, green and orangish, included mud, sand, clay, tiny rocks, some leaves |  |  |  |  |  |  |  |  |  |
| Sediment Core |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| 8 cm | No |  | No |  | No |  | Rare: Gravel Common: Clay, Sand Abundant: Mud | Rare: Leaves |  |
| Boats-NR |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial | 10:09 |  | Tugboat |  |  |  |  |  |  |
| Residential | 10:38 |  | Motorboat |  |  |  |  |  |  |