Day in the Life of the Hudson River 10/14/21

Peebles Island State Park

RIVER MILE: 154

Salt Front: ~ 49

D. Lee Magadini & Berkshire Waldorf High School

12 students (9th-12th grade) & 2 adults

Latitude 42.78 N & Longitude – 73.68 W

Location: Peebles Island State Park

Area: A large grassy riverside park with many trees surrounding an open pavilion with picnic tables inside and an overlook that has 180° view of the Hudson River.

Surrounding Land Use: 55% Urban/Residential; 20% Forested; 5% Industrial/Commercial; 10% Beach; 10% Roadways and Bridges.

Sampling Site: Forested, grassy, covered in vegetation, parking lot. A beach with altered banks that have stairs to the water.

Shoreline: 10% Sandy, 80% Muddy, 10% Rocky

Water: Calm, estimated depth of 25 cm.

Plants in area:  25% Eurasian water milfoil; 5% Willow

River Bottom: Muddy, rocky and weedy

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature |  | 11:37 AM 12:31 PM 1:34 PM |  | 21.128 o C 22.24 o C 22.24 o C |  |  | 70 o F 72 o F 72 o F |  |  |
| Wind Speed: |  | 11:25 AM |  | Anemometer 2 mph |  |  | Beaufort 1 | Direction SW |  |
| Weather today |  | 9:55 AM |  | 51-75% cloud cover |  |  | No rain |  |  |
| Weather the past 3 days: |  | Unseasonally warm and clear |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Height (cm) |  |  |
| Above the head of tide |  | 11:35 AM-12:00 PM |  | Unchanged |  |  | 25.5 cm |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Slack |  |
| Rate of Flow |  | 11:44 AM 11:53 AM 12:22 PM 12:30 PM |  | 6.9 0 4.5 6.7 |  | 0.134 0 0.0914 0.13 | South NA NR NR | NA NA NA NA |  |
| Water Temperature- |  | 11:35 AM 1:38 PM |  | 19.4o C 20.02 o C |  |  | 66.92o F 68 o F |  |  |
| Turbidity- Long Sight Tube |  | 11:14 AM 11:53 AM 2:11 PM |  | 60.4 cm 52.8 cm 66.32 cm |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity- Low Range Quantabs |  | 11:12 AM 12:35 PM 2:07 PM |  | 37 ppm Cl- 28 ppm 37 ppm |  |  | 67 ppm TS *Below Detection 67 ppm TS | Any salinity would have been background from geology or road salt |  |
| Standardized Salinity- Low Range Quantabs |  | 9:50 AM |  | BD |  |  |  |  |  |
| DO – Hach Drop Count Kit |  | Time |  | Temp |  |  | ppm DO | % Saturation |  |
|  |  | 11:35 AM 1:38 PM |  | 19.4o C 20.02 o C |  |  | 10 12 | 103% 130% |  |
| pH – Color Match Kit |  | 11:26 AM 12:35 PM 1:53 PM |  | 7.5 8.0 7.5 |  |  |  |  |  |
| Nitrates- NR |  |  |  |  |  |  |  |  |  |
| Phosphates- NR |  |  |  |  |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |  |
| Fish - |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Seine – 5 pulls |  | 17 66 1 1 4 |  | Spottail Shiner Banded Killifish Brook Silverside Smallmouth Bass Tessellated Darter |  |  | 6 cm 10.5 cm 6 cm 7 cm 5 cm | NR 4 cm NA NA NR |  |
| Fish Totals |  | TOTAL: 89 |  | Diversity: 5 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Macroinvertebrate Totals |  | TOTAL: 0 |  | Diversity: 0 |  |  |  |  |  |
| Sediment Core - NR |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| Boats |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial- None |  |  |  |  |  |  |  |  |  |
| Residential | 10:41 AM 10:55 AM 12:31 PM 2:11 PM |  | Unnamed Seaplane NR Starcraft- small fishing boat Starcraft |  | White Gray & Black Brown Gray & Black |  | Southbound Southbound Southbound Northbound | Light Light Light Light |  |