Day in the Life of the Hudson River 10/14/21

Sarah Lawrence College CURB: Beczak, Yonkers

RIVER MILE: 18

Salt Front: ~ 49

Eli Caref & Sarach Lawarence College Center for Urban River at Beczak

2 students & 3 adults

Latitude 40.938483 N & Longitude -73.90291 W

Location: Sarah Lawrence College CURB at Beczak, Habirshaw County Park, Yonkers, Westchester County, NY

Area: 2 acre riverfront county park covered mostly by a grass lawn leading down to the river where it meets a tidal wetland and sandy beach. Property used for fishing, boats, seining, field trips and some education programs.

Surrounding Land Use: 75% Urban/Residential; 25% Beach/Waterfront

Sampling Site: Sandy beach, tidal marsh area covered with vegetation, banks altered, large riprap on shoreline, collected wood/debris in area

Plants in area: No plants in the water area; however in the marsh, there is mud wort, reed grass, salter cordgrass, big cordgrass

River Bottom: Muddy

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature |  | 10:33 AM |  | 21.1 o C |  |  | 70 o F |  |  |
| Wind Speed: |  | 10:33 AM |  | Anemometer NR |  |  | Beaufort 1 | Direction NW |  |
| Weather today |  | 10:33 AM |  | <25% cloud cover |  |  | No Rain |  |  |
| Weather the past 3 days: |  | NR |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Height (cm) |  |  |
|  |  | 10:33 AM 11:22 AM |  | Falling Rising – see note! |  |  | 12 cm 12 cm | *LT was at 11:01 AM so sampled on either side of the falling and rising tide! |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |  |
|  |  | 10:40 AM |  | 15 |  | 0.29 | South | Ebb |  |
| Water Temperature- |  | 10:57 AM |  | 20.5o C |  |  | 68.9o F |  |  |
| Turbidity- YSI probe |  | 10:57 AM |  | 74.8 cm |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity- Meter/probe |  | 10:57 AM |  | 6.65 ppt |  |  |  |  |  |
| Standardized Salinity – HR Quantab |  | 10:57 AM |  | 6,256 ppm (6.3 ppt) |  |  |  |  |  |
| DO – meter/prob |  | Time |  | Temp |  |  | ppm DO | % saturation |  |
|  |  | 10:57 AM |  | 20.5 o C |  |  | 6.72 | 70% |  |
| pH – meter |  | 10:57 AM |  | 7.51 |  |  |  |  |  |
| Nitrates- NR |  |  |  |  |  |  |  |  |  |
| Phosphates- NR |  |  |  |  |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |  |
| Fish – seine 11:05 AM |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
|  |  | 1 1 1 1 |  | Bay Anchovy Atlantic Silverside White Perch Striped Bass |  |  | 6.5 cm 1.7 cm 18 cm 19 cm |  |  |
| Fish Totals |  | TOTAL: 4 |  | Diversity: 4 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
|  |  | 8 1 |  | Blue Crab Crangon Shrimp (Sand) |  |  | 8 cm 4 cm | NR |  |
| Macroinvertebrate Totals |  | TOTAL: 9 |  | Diversity: 2 |  |  |  |  |  |
| Sediment Core |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| 24.13 cm | Yes (mild smell) |  | No |  | No |  | Abundant: clay, mud, sand Common: pebbles, leaves | Rare: wood |  |
| Boats-NR |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial |  |  |  |  |  |  |  |  |  |
| Residential |  |  |  |  |  |  |  |  |  |