Day in the Life of the Hudson River 10/14/21

HUDSON RIVER PARK PIER 40, MANHATTAN

RIVER MILE: 2.9

Salt Front: ~ 49

30 students 10th grade & 5 adults

Toland Kister, Hudson River Park River Project

Scott Wassmuth, Museum School, New York City

Latitude 40.72842 N & Longitude – 74.01379 W

Location:  Hudson River Park, Pier 40 (also called North River), Manhattan, NY

Area: Off southern side of Pier 40 about 600-800 feet down the pier from shore. Large urban pier with many sports facilities: athletic fields, kayaking and boathouse, seasonal moorings available, parking, fishing on the pier, benches and picnic tables for the public

Surrounding Land Use: 50% Urban/Residential; 50% Industrial/Commercial

Sampling Site: Pier, Bulkhead, banks altered; concrete and wood piling, large pier extends into river/cement bulkhead south ~100 m

Shoreline: Rocky

River Bottom: Soft sediment habitat, muddy

Plants in area: no plants in water area

Water: choppy, 15-20 ft

NOTE: this highly urban site is just west of the busy Westside Highway- it is the largest pier in Hudson River Park with a 14 acre footprint; Pier 40 is used for recreation, school groups using the athletic fields; there are 3 public boathouses; there are many people by the river’s edge jogging, biking, and walking.

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature |  | 10:30 AM |  | 18.4 o C |  |  | 65 o F |  |  |
| Wind Speed: |  | N/A |  | N/A |  |  |  |  |  |
| Weather today |  | 10:30 AM |  | 26-50% cloud cover |  |  | No Rain |  |  |
| Weather the past 3 days: |  | Fairly clear, warm, some light precipitation at night |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Height (cm) |  |  |
| Measured change in depth of water at pier |  | 10:28 AM 11:03 AM |  | Rising Rising |  |  | 328.3 361.3 |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |  |
|  |  | N/A |  | N/A |  | N/A | N/A | N/A |  |
| Water Temperature- |  | 10:36 AM |  | 22.5 o C |  |  | 72.5 o F |  |  |
| Turbidity- Long sight tube |  | 10:30 AM 10:45 AM 10:55 AM |  | 39 cm 45 cm 53 cm |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity- Students - Hydrometer |  | 10:33 AM |  | 13.6 ppt |  |  | Average reading |  |  |
| Salinity Standardized - Refractometer |  | 12:30 PM |  | 14 ppt |  |  |  |  |  |
| DO – chemets |  | Time |  | Temp |  |  | ppm DO | % saturation |  |
|  |  | 10:40 AM |  | 22o C |  |  | 6 ppm | 70% |  |
| pH – color match |  | 10:30 AM |  | 7 |  |  |  |  |  |
| Nitrates- |  | N/A |  | N/A |  |  |  |  |  |
| Phosphates- |  | N/A |  | N/A |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |  |
| Fish |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
|  |  | 1 1 |  | Black sea bass Oyster toadfish |  |  | 4.75 cm 8 cm |  |  |
| Fish Totals |  | TOTAL: 2 |  | Diversity: 2 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
|  |  | 2 16 |  | Blue Crab Isopods (Idotea metallica) |  |  | 11 cm 1.2 cm | < 1 cm < 1 cm |  |
| Macroinvertebrate Totals |  | TOTAL: 18 |  | Diversity: 2 |  |  |  |  |  |
| Sediment Core |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| N/A | N/A |  | N/A |  | N/A |  | N/A | N/A |  |
| Boats |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial | 10:56 AM |  | NY Waterway |  | White |  | North |  |  |
|  | 10:57 AM |  | Ferry |  | White/Blue |  | North |  |  |
| Residential |  |  |  |  |  |  |  |  |  |