Day in the Life of the Hudson River 10/14/21

BOWLINE POINT TOWN PARK, HAVERSTRAW

RIVER MILE: 37

Salt Front: ~ RM 49

North Rockland High School- Jeanne Maguire, C. Galligan and 4 teachers & 4 volunteers

77 students, 11-12th grade

Latitude 41.20314 N & Longitude – 73.95765 W

Location: Bowline Point Town Park, Haverstraw, Rockland County, NY

Area: Recreation, fishing area

Sampling Site: Beach area, pier, riprap, brick pieces

Surrounding Land Use: 10% Urban/Residential, 20% Forested, 10% Beach, 40% Industrial/Commercial, 20% Picnic/Park

Shoreline: 20% Sandy, 80% Rocky

Water: 91.4 cm depth

River Bottom: 20% Sandy, 80% Rocky

Plants in water area: None

| ITEM |  | Time |  | Reading |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature |  | 9:19 AM 10:18 AM |  | 21 o C 29.5 o C |  |  | 70 o F 78 o F |  |  |
| Wind Speed: |  |  |  | 1 m/sec |  |  | 2.24 mph |  | North |
| Weather today |  | Clear, no rain |  |  |  |  |  |  |  |
| Weather the past 3 days: |  | Rained on 10/12/21 |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Height (cm) |  | Rate of change (cm/min) |
|  |  | 9:58 AM 10:13 AM 10:42 AM |  | - Falling Falling |  |  | 73 cm 68 cm 63 cm |  | - 0.33 cm/min 5.8 cm/min |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South |  | Ebb/Flood/Still |
|  |  | 9:09 AM 10:00 AM 10:45 AM |  | 30 4.7 57.5 |  | 0.584 0.091 1.112 | South South South |  | Ebb Ebb Ebb |
| Water Temperature- |  | 9:20 AM 10:00 AM 12:04 PM |  | 21, 21.7, 23, 23, 24 22.5 o C |  |  | 70, 71, 73.4 73.4, 75.2 72.7 o F |  | Avg: 21.9 o C/71.5 oF Avg: 23.5 o C/74.3 oF Avg: 22.5o C/72.7 oF |
| Turbidity- |  |  |  | Secchi Disk 61cm, 56cm, 71cm: Avg: 62.7cm |  |  |  | Long Sight Tube 25cm, 27cm, Avg: 26 cm |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity- Low Range Quantabs |  | 10:22 AM |  | 381 ppm Cl-, 301 ppm Cl- Avg: 341 ppm Cl- |  |  | 616 ppm total salinity |  |  |
| DO – Drop count kit |  | Time |  | Temp |  |  | ppm DO |  | % saturation |
|  |  | 10:07 AM 11:00 AM |  | 24 o C 21 o C |  |  | 7 ppm 8 ppm |  | 80% 85% |
| pH – Color match kit |  | 9:45 AM - 10:38 AM |  | 7.5, 7.5, 7.5, 8.0, |  |  |  |  | Avg: 7.6 |
| Biological |  |  |  |  |  |  |  |  |  |
| Fish |  | Number |  | Name |  |  | Length of longest |  | Length of smallest |
| 23ft net 18 seines 9:20-12:00 PM |  | 47 4 1 1 |  | Atlantic Silversides Spottail Shiner American Shad Striped Bass |  |  | 7 cm - - 20.32cm |  |  |
| Fish Totals |  | TOTAL: 53 |  | Diversity: 4 |  |  |  |  |  |
| Boats |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound |  | Light/Loaded |
| Commercial | 9:04 AM 9:20 AM 9:20 AM |  | Ferry Cargo Cargo |  | White White/Black Black |  | West to dock North North |  | Loaded Loaded Loaded |
| Recreational | 9:05 AM 9:20 AM 10:35 AM 11:19 AM 11:22 AM |  | Yacht Sail Sail Jetski Boat |  | White/Black White Red White Black/White |  | South South South South North |  | Light Light Light Light Light |