Day in the Life of the Hudson River 10/14/21

Norrie Point Environmental Center

RIVER MILE: 84.5

Salt Front: ~ 49

Maija Niemisto, NYSDEC & FDR High School

10 students & 4 adults

Latitude 41° 49’ 55’’ N & Longitude – 73° 56’ 32’’ W

Location:  Norrie Point Environmental Center, Norrie Point, Dutchess County, NY

Area: East side of the Hudson, far from the main channel in a protected cove. Substantial paved areas nearby, large environmental center building adjacent to the water. Shallow cove facing south onto the Hudson River, bordered by riprap embankment, dock

Surrounding Land Use: 90% Forested; 10% Residential

Sampling Site: Front banks altered with riprap, cove is very muddy with some riprap and the west side is rocky

Shoreline: beach area, shore with bulkhead, shoreline riprap, banks altered for DEC building, riprap along the railroad tracks, piping entering river area

River Bottom: 50% muddy, 50% rocky

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature |  | 10:00 AM |  | 22.1 o C |  |  | 71.78o F |  |  |
| Wind Speed: |  | 10:00 AM |  | Anemometer 1.0 m/s |  |  | Beaufort NR | Direction S |  |
| Weather today |  | 9:00 AM |  | 26-50% cloud cover |  |  | No Rain |  |  |
| Weather the past 3 days: |  | Foggy but no rain Warm, wind & foggy (in the mornings) |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Height (cm) |  |  |
|  |  | 9:00 AM 9:57 AM 11:06 AM |  | Falling Falling Falling |  |  | 139.7 cm 133.35 cm 106.68 cm |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |  |
|  |  | 10:30 AM |  | 1.94 |  | 0.0377 | South | Ebb |  |
| Water Temperature- |  | 12:00 PM |  | 19.5o C |  |  | 67.1o F |  |  |
| Turbidity- Long Sight Tube |  | 12:22 PM 3:50 PM |  | 39 cm 23.5 cm |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity- Hydrometer Refractometer |  | 12:04 PM |  | 0 ppm |  |  | *Below Detection for these methods |  |  |
| Standardized Salinity – LR Quantabs |  | 12:10 PM |  | 76 ppm |  |  |  |  |  |
| DO – Hach Drop Kit |  | Time |  | Temp |  |  | ppm DO | % saturation |  |
|  |  | 12:15 PM |  | 19.5o C |  |  | 9 ppm | 100% |  |
| pH – color match kit |  | 12:15 PM |  | 7 |  |  |  |  |  |
| Nitrates- NR |  |  |  |  |  |  |  |  |  |
| Phosphates-NR |  |  |  |  |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |  |
| Fish - |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Seined 6 times |  | 1 11 18 6 1 12 28 11 1 7 |  | American Eel Herring sp. Spottail Shiner Banded Killifish White Perch Redbreast Sunfish Pumpkinseed Bluegill Largemouth Bass Tessellated Darter |  |  | NR | NR |  |
| Fish Totals |  | TOTAL: 96 |  | Diversity: 10 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
|  |  | 1 |  | Blue Crab |  |  | NR | NR |  |
| Macroinvertebrate Totals |  | TOTAL: 1 |  | Diversity: 1 |  |  |  |  |  |
| Sediment Core |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| 13.5 cm | No |  | No |  | No |  | Abundant: mud, sand Common: clay, gravel Rare: pebbles | Common: leaves Rare: wood |  |
| Boats |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial | 9:50 AM 10:50 AM |  | Hamilton Ann |  | White/YellowRed/Black |  | Southbound Northbound | Light Loaded |  |
| Residential - NR |  |  |  |  |  |  |  |  |  |