Day in the Life of the Hudson River 10/14/21

Norrie Point Environmental Center

RIVER MILE: 84.5

Salt Front: ~ 49

Maija Niemisto, NYSDEC & Susan Hereth & Kingston Farm Group

9 students & 3 adults

Latitude 41° 49’ 55’’ N & Longitude – 73° 56’ 32’’ W

Location:  Norrie Point Environmental Center, Norrie Point, Dutchess County, NY

Area: East side of the Hudson, far from the main channel in a protected cove. Substantial paved areas nearby, large environmental center building adjacent to the water. Shallow cove facing south onto the Hudson River, bordered by riprap embankment, dock

Surrounding Land Use: 90% Forested; 10% Residential

Sampling Site: Front banks altered with riprap, cove is very muddy with some riprap and the west side is rocky

Shoreline: beach area, shore with bulkhead, shoreline riprap, banks altered for DEC building, riprap along the railroad tracks, piping entering river area

River Bottom: 50% muddy, 50% rocky

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature |  | 3:38 PM |  | 27 o C |  |  | 80.6 o F |  |  |
| Wind Speed: |  | 3:38 PM |  | Anemometer 0.9 m/s |  |  | Beaufort NR | Direction SW |  |
| Weather today |  | 3:40 PM |  | >75% cloud cover |  |  | No Rain |  |  |
| Weather the past 3 days: |  | Foggy but no rain Warm, wind & foggy (in the mornings) |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Height (cm) |  |  |
|  |  | 3:33PM-4:15 PM |  | Slack Falling |  |  | 36 cm 28 cm |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |  |
|  |  | NR |  |  |  |  |  |  |  |
| Water Temperature- |  | 4:09 PM |  | 12 o C |  |  | 53.6 o F |  |  |
| Turbidity- Long Sight Tube |  | 3:50 PM |  | 23.5 cm |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity- Hydrometer Refractometer |  | 4:05 PM |  | 0 ppm |  |  | *Below Detection |  |  |
| Standardized Salinity – LR Quantabs |  | NR |  |  |  |  |  |  |  |
| DO – Hach Drop Kit |  | Time |  | Temp |  |  | ppm DO | % saturation |  |
|  |  | 4:14PM |  | 12 o C |  |  | 11 ppm | 105% |  |
| pH – color match kit |  | 3:58 PM |  | 7.5 |  |  |  |  |  |
| Nitrates- NR |  |  |  |  |  |  |  |  |  |
| Phosphates-NR |  |  |  |  |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |  |
| Fish - |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Seined 2 times |  | 7 5 20 9 2 17 10 1 3 |  | American Shad Golden Shiner Spottail Shiner Banded Killifish Redbreast Sunfish Pumpkinseed Bluegill Smallmouth Bass Tessellated Darter |  |  | NR | NR |  |
| Fish Totals |  | TOTAL: 172 |  | Diversity: 13 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
|  |  | 2 |  | Blue Crab |  |  | NR | NR |  |
| Macroinvertebrate Totals |  | TOTAL: 2 |  | Diversity: 1 |  |  |  |  |  |
| Sediment Core |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| NR |  |  |  |  |  |  |  |  |  |
| Boats |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial |  |  |  |  |  |  |  |  |  |
| Residential - NR |  |  |  |  |  |  |  |  |  |