Educator Packet for A Day in the Life of the Hudson & Harbor

http://www.ldeo.columbia.edu/dayinthelife

The Packet is designed for educators & teachers with information on a range of data gathering activities that are a part of A Day in the Life of the Hudson and Harbor.

Please Note:

- Any combination of these activities can be completed as part of the day’s events.

- Additional activities are available on the Day in the Life website.

- STUDENT DATA RECORDING SHEETS are available on the website and should be used for recording data for the event. After recording we ask that all data be submitted within 24-48 hrs through an online data collection form that we will provide annually, along with photo scans of the student forms. In this packet we have included examples of data collection for each parameter outline in blue below.

PLEASE BE SURE TO RECORD TIME & UNITS OF MEASURE FOR EACH SAMPLING ITEM SO THAT COMPARISONS CAN BE MADE THROUGHOUT THE RIVER.

The topics covered here correspond with the Student Packets

| ACTIVITY | PAGE |
| --- | --- |
| COVER PAGE: Site Information - SURROUNDINGS DATA: - Location, River Mile, Longitude & Latitude - Physical Survey - Sketch - Boats & Ships - PHYSICAL RIVER: - Tides & Currents - Weather & Wind - CHEMISTRY of the RIVER: - Water Temperature - Salinity - pH - Alkalinity, Nitrates, Phosphates - Dissolved Oxygen - TURBIDITY - SEDIMENT CORE - FISH & MACROINVERTEBRATES - Hudson River Watershed Fish Fauna Checklist - JOURNALING & HUDSON RIVER ALMANAC | Page 1-4 Page 5-8 Page 9-11 Page 12 Page 13-14 Page 15-17 Page 18 |

COVER PAGE: SITE INFORMATION

Site information provides us with record of key information about who collected data from this site during this year (i.e. coordinator, organization, school, address, contact information, etc).

Teachers or educators can help the students complete this section.

ACTIVITY I- SURROUNDINGS DATA

Physical Survey: One of the many ways that the Hudson River is dynamic is from the ever changing and developing shoreline and even plant material in the water. A sampling site can change subtly or drastically from year to year. The physical characteristics of a site can influence the turbidity, chemistry, habitat function and biology that we find. We want to make sure that the data captures these changes so it is important for your students to take a step back and first observe their surroundings!

1. LOCATION, RIVER MILE, LATITUDE & LONGITUDE

The students’ data is a part of a large effort across 100 sites along the Hudson River and neighboring tributaries. Students should recognize the importance of recording the site information that is requested on the form as each is critical to the long-term value of the data:

- Location

  - Site name, City, County, State

- River mile

  - If you don’t know your river mile, you can use the Hudson River Estuary Map (page 2) to help identify a general location. Note that when we post the data we will use the RM we have assigned it from our database of overall sites.

- GPS Latitude & Longitude

  - We use the latitude and longitude for site location. This can be obtained from a GPS, or a compass app on a smart phone (location must be enable) or Google Maps/Maps on a smart phone.

  - This is also an educational activity for the students to help them to develop geospatial skills through connecting these numbers to a location on a map.

    - First: Use the latitude and longitude from the step above.

    - Second: Use a river map with latitude and longitudinal markings along the edges, this can be a navigational charts or this simple river map can be printed and taped together into one long map for used. In this activity students locate the latitude and longitude along the edge of the map and draw perpendicular lines (actual or imaginary) to connect the two locations. This should match where you are!

2. AREA

In one sentence, the students are to describe their area so someone could find their sampling site. Be specific! Students should specify:

- Their location in relation to a significant landmark (i.e. bridge or pier)

- Whether they are sampling within a cove, on the beach, off the pier, etc.

- How is this site used by people? Do people come for picnics, launching boats, fishing, swimming, or other activities?

3. SAMPLING SITE

Make observations about your sampling site. Ensure that your students check all that apply.

- Is there a pier?

- Is it forested?

- Is it open and grassy?

- Is there a parking lot?

- Is there a piping entering the water

- Are the banks altered?

- Is it lined with bulkhead- seawalls, wooden timbers, or metal plates to hold the shore in place

- RipRap- a line of large rocks piled up on shore

All of these features can impact the data that you collect. Teachers can lead a discussion with their students about how each one of these features could impact their data.

4. SURROUNDING LAND USE

Make observations about the surrounding land use. How the neighboring land is being utilized can also impact the data you collect. When deciding the size of the area to include you should consider what you can see in the area around you.

Record the surrounding land uses as a percentage of 100%. For example, 50% is half the usage.

- % Urban/Residential

- % Forested

- % Beach

- % Industrial/Commercial

- % Other (specify)

5. SHORELINE

Describe the shoreline sediment:

Record the shoreline as a percentage of 100%. For example, 50% is half usage.

- Sandy

- Muddy

- Rocky

Note: Other descriptions of the shoreline alterations should be included in the Sampling Site description.

6. RIVER BOTTOM

Describe the water area itself and the composition of the bottom of the river. The water area and bottom type can influence the different biological species that we find.

Students should record:

- The estimated Water Depth (cm)

- Check all that apply:

  - Bottom muddy

  - Bottom sandy

  - Bottom weedy

  - Bottom rocky

7. WATER

The wind speed can impact the physical and chemical conditions of the water. On the surface of the water, observe whether the water is Calm or Choppy.

8. PLANTS IN WATER

Aquatic and submerged aquatic vegetation provide fish habitat, filter out sediments and nutrients, and can assist with oxygen exchange in the water. Some species, like water celery, add dissolved oxygen to the water. Other species, like water chestnuts, can create an oxygen-depleted zone. Some plants are native, while others are non-native or invasive. In the freshwater section of the river you can use the Hudson River Field Guide to Plants of Freshwater Tidal Wetlands to identify plants found growing in the water.

Record the aquatic plants species as a percentage of the total area covered. For example, if water chestnuts covered half of the site, it would be 50%. If NONE, please check “No plants in water area.” Below is a list of aquatic and submerge vegetation that you may see at your site:

| Tidal Shallows |  |  |  |
| --- | --- | --- | --- |
| Eurasian Water Milfoil | Water Chestnut | Water Celery | Other (Specify) |
| Tidal Marshes |  |  |  |
| Arrow Arum | Arrowhead | Big Cordgrass | Broad Leaved Cattail |
| Bur-Reed | Golden Club | Jewelweed | Mud Wort |
| Narrow-leaved Cattail | Pickerelweed | Phragmites/Common Reed | Purple Loosestrife |
| Reed Grass | Saltwater Cordgrass | Spatterdock | Swamp Rose-Mallow |
| Sweet Flag | Wildrice | Yellow Flag | Yellow Pond Lily |
| Other (Specify) |  |  |  |

9. Sketch the Sample Site:

Students will sketch a map of your sampling site. Include a compass rose (N, S, E, W) and rough scale. Label landmarks or notable features. Indicate specific locations where you sampled. If available, use a digital camera to photograph your site.

Example:

10. Boats and Ships:

The Hudson River is an industrious river, often being used for business and pleasure. You may see boats and large ships, tugs, or barges pass by your site. We are mainly interested in commercial shipping to show the working river and exhibit how the river is being used (i.e. large boats, tugs, or barges). If recreation use dominates the river, we should note this as well (i.e. sailboats, kayaks, speed boats).

Shipping Details: Record the time, name of the vessel, color, direction (North or South), and whether it is loaded or light. A loaded ship or barge is full of cargo, and rides lower in the water than a light, empty vessel. Binoculars are helpful in gathering the information requested here. If details on this are collected it is possible to track a ship’s movement up or down river and even calculate its transit speed!

ACTIVITY II – PHYSICAL RIVER

1. TIDES:

- Tides are the rise and fall of the water level in large bodies of water that are caused by the combined effects of the gravitational forces exerted by the Moon and Sun, and the rotation of the Earth. Through its connection to the Atlantic Ocean the estuary section of the Hudson River experiences semi-diurnal tides or two nearly equal high and low tides a day. Therefore, every 6 hours, the tides will shift. On a flooding tide, salty water from the Atlantic Ocean is pushed into Hudson, causing the water to rise and bringing the leading edge of the salt further North. On an ebbing tide, water is pulled back toward the ocean, lowering the tide and moving the leading edge of the salt South.

- Tidal range is the vertical different in height between the high tide and low tide, which varies depending on where you are in the river and is controlled by the river width. Much of the central section of the river experiences a tidal range ~ 3ft, while at the Troy dam the tidal range is close to 5 ft. The vertical tidal range can also vary with different stages of the moon. During a full moon, when the Earth is between the sun and the moon, and the new moon, when the moon is between the sun and the Earth, the sun and moon alignment increases the gravitational pull. This strong gravitational pull causes very high and very low tides resulting spring tides, or a large tidal range. During the moon’s quarter phases, the gravitational pulls cancel out, and the tidal ranges are at their lowest known as neap tides.

Method 1: Tide Stick Method

- Firmly set a dowel marked in 10cm increments into the river bottom sediment in the water & leave it.

- Once set the students record where the water level is on the stick as the “Start Time-Water Height (cm)”

- Students check the tide stick every 15-30 minutes to record the water height on the dowel and the corresponding time. Students can observe (or even calculate) how quickly the tide changes throughout the day.

- As the tide rises (flood tide), the measurements on the tide stick will be increasing as the water moves up the stick. As the tide falls (ebb tide), the measurements will get smaller.

- Be sure you talk this through with your students so you are sure they understand so they correctly record whether the tide is ebbing, flooding, or still/slack tide, when the tide is transitioning and no change in measurement is recorded.

Method 2: Pier/Dock to Water

- The goal is to measure from the dock to the surface of the water. Tape measures (cm) are fairly light-weight so this might need to be done with a weighted line that is then measured with a tape.

- Mark your location on the dock so measurements are collected from the same place each time.

- Students remeasure every 15-30 minutes and record the distance between the dock and the surface of the water and the corresponding time. Students can observe (or even calculate) how quickly the tide changes throughout the day.

- Since you are measuring the dock to the surface of the water, as the tide rises (flood tide), the measurements will get smaller.  As the tide falls (ebb tide), the measurements will get larger.

- Be sure you talk this through with your students so you are sure they understand so they correctly record whether the tide is ebbing, flooding, or still/slack tide, when the tide is transitioning and no change in measurement is recorded.

Simple Method:

- At a beach- Use slender strong sticks as tide markers. At the start, place one stick at the water’s edge. Push it deep into the ground or pile rocks at its base to hold it in place.  Every 15 or 30 minutes check your marker. If the level has changed, place the second stick to mark the new position of the water’s edge on a beach so through time you see the total change.  Record on your sheet the time and whether the water level was flooding, ebbing, or still.

- Pier or bulkhead- Choose a distinct, immoveable feature on or near the bulkhead to see whether the water level is rising or falling. If there are waves, use your judgment in deciding where the water’s edge/surface is. Record on your sheet the time and your observations. After several observations record whether the water level was flooding, ebbing, or still.

**Extra Activity:

If your students have time at this station, they can calculate how quickly the tide is rising or falling by dividing the change in height by the time between recordings. Think of the basic definition of speed as distance divided by time  (D)/T.

Calculation:

- Subtract a time from prior reading from the time of this reading for ‘time of travel’ (or time elapsed).

- Calculate the change in height by subtracting the water height from the prior reading from the water height from this reading for the ‘distance traveled’.

- Divide the change in height by the time elapsed. This is the rate (speed) of tidal change.

(Water Height (cm) – Start Height (cm))/ (Checked Time – Start Time)= Speed of Tidal Change (cm/min)

2. CURRENTS

Currents are the internal movement in the water sometimes described as a push and pull in the water. The Hudson River is known as the ‘river that flows both ways’ because it’s currents are driven by the rising and falling of tides, mimicking the tide’s ~6 hour cycle. However, extreme weather, such as excessive rains or strong winds, can impact the currents by overwhelming or suppressing them.

- BE CAREFUL! The tidal current does not always coincide with the wind blown surface water. While the water may look as if it is flowing in one direction, the current’s direction may surprise you!

- NOTE: Make notes of any features about the site, river or shoreline that may cause the current near shore to flow in a different direction than the current out in the middle of the Hudson (a protected embayment, a pier jutting out causing an unusual swirling)?

- DATA FROM THE MAIN CHANNEL IS THE BEST DATA TO RECORD SO PLEASE BE ALERT TO DIFFERENCES THAT MIGHT EXIST AND MAKE A NOTE.

Method 1: Orange Toss

- This is a great teamwork activity! This measurement requires full participation from 5 students.

- Record the time that you start collecting current data. Remember currents vary throughout the day!

- Make a Prediction- what way do students predict the current is moving?

- Measure 10m and lay out the tape measure on the ground.

- Student #1 and #2 should be position at 0m and 10m (i.e. at start and end of the measuring tape). Have them hold up a clipboard or binder to block their vision from the orange being tossed in the middle of the measuring tape. Additionally it helps to close the eye furthest from the clipboard so they can only see straight ahead.

- Student #3 will toss an orange at the 5m marker (middle) as far as they can, trying to reach the main current of the river.

  - An orange is used to measure the speed of the current because oranges have the perfect buoyancy to float and still be heavy enough to rest in the tidally driven current, not the wind driven surface waves. A large solid stick that is large enough so the wind can’t push it can work as well.

- When the orange hits the water, it will submerge and then pop back to the surface. The moment that the orange resurfaces, Student #4 should start the stopwatch.

- Often the orange is not thrown straight so Student #5 should mark the starting point of the orange if it is not perfectly aligned with the middle of the tape measure (5m).

- Once the orange is parallel to one end of the tape measure (either 0m or 10m depending on whether it is flowing north or south), either Student #1 or #2 will yell “STOP” when they see the orange come into their sight while still holding the “blinder”.

- Student #4 will immediately stop the stopwatch.

- Calculate & Record:

  - Record the time that your team started collecting current data

  - Measure the distance travelled by the orange using the starting position of where the range entered the water (Student #5) to the end of the distance traveled. We will work in cms.

  - Record the amount of time in seconds that it took the orange to travel the measured distance

  - Calculate the current speed by dividing the distance by time (cm/sec).

  - If the current is Ebbing (outgoing/south), Flooding (incoming/north), or Still

  - Record any Metadata or observations about your site that could influence the currents.

Method 2: Simple Method

Every student rotation or every hour toss an orange (or stick) as far out into the water as you can. Record the time and direction of travel as Flooding (North towards Albany), Ebbing (South towards the Atlantic), or still (slack tide is in between the high and low tide).

Extra Activity: Calculate KNOTS: If your students want to calculate the rate of travel in knots use the distance in cm for 60 seconds to compute this.  Let’s think this through:

1 kt. = 6076 ft. per hr. Convert ft. to cm.  1 ft. = 30.48 cm. so multiple these two to compute cm/hr or 185196.5 cm/hr. Now divide by 60 for cm per minute (3086.6 cm/min.) now by 60 again for cm/sec.  What you find is that: 1 kt =51.44 cm/sec.

SO to compute Knots from cm/sec use the following equation:

Knots= (cm/sec)/51.4

Example:  If the orange travelled 63 cms in 30 seconds divide 63/30 = 2 cm sec. /51.4 = .04 kts.

WEATHER AND WIND:

Weather and wind are important pieces of physical data that help to provide context for the other data. Weather includes current conditions and conditions over the last few days that may have an impact on the data you collect today (such as rain, extremely hot or cold temperature). Wind levels can increase choppiness in the water thus adding oxygen and increasing levels of oxygen saturation. Wind can also affect movement on the top of the water surface making it difficult to assess currents.

- AIR TEMPERATURE

Conversions:			◦C   =   0.556 X (◦F  -  32◦ )

◦F    = (1.8  X ◦C ) + 32◦

Record conditions at the start of sampling. Record changes every hour if possible.

Time_10:30AM 	     Air temperature  		56	o F    	13.3	o C

Time	11:30 PM 	     Air temperature  		59	o F    	15	o C

- WIND SPEED

(PLEASE REFER TO BEAUFORT CHART BELOW)

Using the Beaufort chart record the FIRST COLUMN as Beaufort FORCE 	3	.

Using an anemometer, record wind speed:  	8	kts (preferred)   and/or ________mph

Record wind direction (direction the wind is coming/blowing from): 		Northeast

- CLOUD COVER

Cloud cover (check one)     clear_______     partly cloudy___     mostly cloudy: X	     overcast___

- RAIN (PRECIPITATION) TODAY & WEATHER FOR THE PAST 3 DAYS

Recent weather (especially rain) can also play an important role in the status of the Hudson River. For example, if we had a large rain storm, it could take days for all the freshwater from the neighboring tributaries to drain into the Hudson River. This ultimately drives the salinity down days after the rain event.

Time: 	12:00 PM    Precipitation? NO How much? __________

If the weather changes over the time you are sampling, please note that here.

The weather became more windy throughout the day.

Briefly describe the weather for the last 3 days. Any rain, wind, or unusual temperatures?

It rain about 3 inches in the past 3 days, and the weather was chilly and windy

ACTIVITY III- CHEMISTRY of the RIVER

1. WATER TEMPERATURE

Water temperature is important for understanding the amount of dissolved oxygen the water can hold, and for the fish communities in the area. Students will better understand Fahrenheit temperatures, but in science it is important to become familiar with Celsius. If possible, record water temperature in BOTH degrees Celsius and degrees Fahrenheit to help them develop a connection between the two. If you don’t have both oC  and  oF thermometers, students can convert between the two using the following formulas:

oC   =   0.556  x  ( oF  -  32)                    oF   =   (1.8  x  oC)  +  32

It is best to take the water temperature several times in succession and then average. Over the day, you will want to check for change, especially in shallow water and backwater areas, which may show more variation through the day due to sunlight, tide or current changes.

Time		Reading 1		Reading 2		Reading 3		Average

10:30AM	oF 	56 				56				58				56.67 F

oC 	13.333			13.333			14.444			13.70 C

2. SALINITY

Different instruments measure salinity as ‘total salts’, or ‘chloride’ (a fraction of total salts) or even conductivity.  Therefore, it is essential that you mark down what instrument you use to measure the salinity. Marine and Brackish sections we use hydrometers, refractometers or meters in ppt or ppm. Freshwater sections of the river, the units of measurement may be parts per million (ppm). The quantabs measure ppm Cl- and not total salinity so please record them that way.

| How do we determine salinity in the Hudson |  |  |
| --- | --- | --- |
| Freshwater <100 ppm | Brackish water 100-18,000 ppm | Marine >18,000-35,000 ppm |

In saltier parts of the estuary, you may also see measurements expressed in ‘parts per thousand’ (ppt); one part per thousand equals 1000 mg/l. In the fresher parts of the estuary background levels are measured using quantabs as Cl – a part of total salinity.

Quantab Strips (PPM Cl-): High Range   X Low Range

Record PPM CHLORIDE  (3rd column on Quantab bottle)

Meter/Probe

(PPT)

Refractometer

(PPT)

Hydrometer

(PPT)

BE SURE TO MARK THE EQUIPMENT YOU ARE USING FOR THE TEST:

Repeat several times in succession and average the results. UNITS, UNITS, UNITS! 

Time		       Reading 1		Reading 2		Reading 3		Average Units

10:30 AM    	848 ppm Cl-      779 ppm Cl-        848 ppm Cl-		825 ppm Cl-

(If reading conductivity please record with appropriate unit uS/cm (microsiemens) or mS/cm (milliseimens and then convert to salinity if you have that ability)

3. pH (POTENTIAL HYDROGEN)

A measure of the acidity of an area. pH has no units listed with it. Neutral is 7, acidic is < 7, basic is >7. Repeat several times in succession and average the results.

BE SURE TO MARK THE EQUIPMENT YOU ARE USING FOR THE TEST:

Test Strips____

Color Match Test Kit  X

01

Meter____

pH Pen____

Time		Reading 1		Reading 2		Reading 3		Average

10:30 AM 		7.5				7.6				8.0 			7.7

ADDITIONAL CHEMICAL TESTS IF DESIRED: 4. Alkalinity, 5. Nitrates, and 6. Phosphates

The following tests typically require more complex methods than those described above. There is no requirement to do these, but if you have the equipment and ability, the data would be welcome

4. ALKALINITY

Alkalinity is a measure of water’s ability to neutralize (buffer) acids, such as those that might be found in acid precipitation. Don’t confuse it with pH.  pH measures how strongly acidic or alkaline the water is; the alkalinity test determines the concentration of alkaline compounds in the water – or water hardness. In pure water, small amounts of acid or alkaline substances will cause dramatic shifts in pH – however with the addition of small particles of water hardness substances in the system causes a buffering that absorbs or soaks up small changes to the system. The expected range of alkalinity in the Hudson is between 80-100 mg/L of calcium carbonate (CaCO3).

Repeat several times in succession and average the results.

Time		Reading 1		Reading 2		Reading 3		Average

11:30 am 		80				93 				85 			86 CaCO3  mg/l

5. NITRATES

Nitrate (NO3- ) is a nutrient used by plants and animals for growth and maintenance. It is relatively plentiful in freshwater ecosystems but less so in saltwater ecosystems, where it is typically the limiting nutrient. The expected range of nitrates in the Hudson River is low (<1-2 mg/L).

If you are measuring with an LaMotte Estuary Wide Sampling kit, you will NOT get an accurate measurement because the range is not high resolution. PLEASE NOTE IF YOU USED THIS KIT.

Repeat several times in succession and average the results.

Time		Reading 1		Reading 2		Reading 3		Average

11:30 am 		1				0.8 				0.85 			0.8833  NO3-(mg/l)

6. PHOSPHATES

Phosphate (PO4-3) enters the water through release by some rocks, soil, and animal waste and serves as a nutrient for plants. It is usually the nutrient least available in freshwater ecosystems. The expected range of phosphates in the Hudson River is low (<1 mg/L).  If you are measuring phosphates with an LaMotte Estuary Wide Sampling kit, you will NOT get an accurate measurement because the range is not high resolution. PLEASE NOTE IF YOU USED THIS KIT.

Repeat several times in succession and average the results.

Time		Reading 1		Reading 2		Reading 3		Average

11:30 am 		1				0.9 				0.83 			0.91  PO4-3 mg/l

7. DISSOLVED OXYGEN (D.O.)

The amount of dissolved oxygen (DO) in a river is one of the most important factors determining its health. Many variables influence DO, including temperature, time of day, presence of plants, and wind conditions. DO measurements are given in mg/l and as percent saturation. At 100% saturation, water of a given temperature cannot hold more DO. If more is added - by wind or turbulence, saturation may temporarily exceed 100%. In this case oxygen will diffuse from the water into the air. Saturation levels below 100% are not necessarily the result of pollution. At night, when plants aren’t producing oxygen through photosynthesis, saturation may fall below 100% as living things use up the available DO.

BE SURE TO MARK THE EQUIPMENT YOU ARE USING FOR THE TEST

Meter/Probe

Drop Count Kit	X

Other

Time		Temperature in oC		DO (mg/l)		% saturation

10:00 AM			16 oC			7 mg/l (or ppm)		 	70%

DETERMINING PERCENT SATURATION THE "QUICK AND EASY" METHOD

Source of chart: http://waterontheweb.org/under/waterquality/oxygen.html

For a quick and easy determination of the percent saturation value for dissolved oxygen at a given temperature, use the saturation chart. Pair up the mg/l of dissolved oxygen you measured and the temperature of the water in degrees C. Draw a straight line between the water temperature and the mg/l of dissolved oxygen. The percent saturation is the value where the line intercepts the saturation scale. Waterways with a saturation value of 90% or above are considered healthy.

ACTIVITY IV: TURBIDITY

Turbidity is water clarity, an important feature of an estuary. In the Hudson River, turbidity is made up of small bits of plankton, pieces of detritus or decomposing plant and animal matter, salt and suspended bits of sediment.  Different techniques for determining turbidity use different units of measurement. Be sure to enter data on the correct line for the technique you use. Repeat several times in succession and average the results.

Be careful! If you are collecting water for a site tube, DO NOT to step in the water as you collect it or you will add turbidity to your sample.

Time		Reading 1		Reading 2		Reading 3		Average

secchi disk		______		________		________		________		________   feet or cm

short sight tube	 ______	       ________		________		________		________    JTUs

long  sight tube  11:00 AM  11.6 cm			12.04 cm 		13.10 cm 		12.25 cm   cm/meters

turbidimeter	 ______	 	________		________		________		________   NTUs

The order of equipment preference is as follows:

1. Long sight tube, 2. Secchi disk, 3. Short sight tube, 4. Turbidimeter.

Observations

How turbid to you think your water is?  How would you describe it in words?

I think the water is very turbid because it is not very clear.

I would describe the water as brown, turbid, and murky.

ACTIVITY V – SEDIMENT SAMPLING

*Use the Step-By-Step directions sheet provided online on the resources page

http://www.ldeo.columbia.edu/edu/k12/snapshotday/Resources.html

BACKGROUND INFORMATION:

- The sediments in the core represent a period of time. The material at the bottom is older than the material on the top.  This is an important principle of geology and much of Earth Science called ‘superposition’.

- If material has been accumulating steadily, a sediment core will contain a record of the material transported by the river through time.

- One of the challenges faced by scientists who study sediment cores is determining the length of time represented by the sediments core. You can not tell how many years your core represents by simply looking at it.  The amount of time represented by your core will vary depending on the specific place and processes of the river in each area.  In sections with high deposition, it could represent a very short amount of time (days to a year), while in other areas it could represent a much longer time (10s to 100s of years or longer).

- What you can tell from looking at a core is whether the color changes over the length of the core. You will note the color of the sediments at the very top of the core. If the color is light brown, this is an indication that the surface sediments are oxidized (in contact with oxygen in the water). The oxidized section is the top represents an unconsolidated recent deposition.  You will measure this and record it to determine how active the deposition is in your area.  Usually, the sediments change to a darker color below the oxidized layer, this is called anoxic (no oxygen) or reducing. It usually means that these sediments have been out of contact with the oxygen in the river water and are older. Often this section will have a sulfur smell noting bacterial decomposition. Extrude your core, then measure and record each section. Complete the core assessment sheet as you observe and describe it. Note anything else that you think is significant. Are there other visible layers? Color changes? Coarse sands and small pebbles collect in areas of fast moving water, fine sands and silts collect in calm and slow water.

- X-Ray Fluorescence (XRF) Spectrometer– What does this mean?  Once you collect your core and describe it you will be sending one to Lamont for X-Ray Fluorescence. This is done with a piece of equipment that can measure lead and other metal concentrations in the sediments.  We focus on lead since it can be used as an indicator of time.  There is a natural background reading of lead in the river (approx. 20ppm) but human (anthropogenic) influences such as early 20th century industry, leaded gas etc. have caused in increase in that level.  Using the XRF we can look at what the readings of various metals are in different areas of the river.  At Lamont we hope is to use this information to roughly constrain (locate the probably range) the age of sediments you collect. A straightforward interpretation of this data is that low levels of lead similar to natural background would indicate sediments that are older than (deposited prior to) approximately 1900, while sediments containing lead a levels elevated above the natural background would indicate sediments that are younger or deposited as part of industrialization.

SEDIMENT SAMPLING:

Push cores are distributed to participating sites where coring is possible. If you don’t have a corer you can skip this activity. Prior to sampling (i.e., before the bag gets wet), please use a permanent marker and label bag with the following information:

- Date – River Site –River Mile

- Example:

- 100809_PP_25 (for Piermont Pier)

- You will be taking a core to examine and describe with your group using the form on the next page. Once the description is complete scoop the pieces into a Ziploc bag and return it to Lamont-Doherty Earth Observatory for X-Ray Fluorescence analysis (this will be picked up). The core will be homogenized for sampling so do not worry about squishing the sample.

DISCUSSION:

Once you have the core for group analysis use the sheet that is in your protocols to look through and analyze it with the group. Discuss how any unusual items might have ended up in the river and the role they play there.

NOTE: if the area you are coring is primarily sand, the corer may not work and the sand may fall out when you lift the corer from the water.  In this case, if you would still like to examine the bottom of the river with your students you might be able to slide a flat piece of something under the base of the corer and still extract a sample to look at.  If that isn’t possible, consider using a jar to scoop out a section trying to obtain a sample that goes down 3-4 inches.  The same activities can be completed with this type of sample. When you bag your sample please note it was not obtained with the corer.

ACTIVITY VI – FISH & MACROINVERTEBRATES

The data section below is set up for fish and invertebrates such as crabs and crayfish that are easily visible without magnification. This sheet can be adapted if you plan to capture and study plankton. If making repeated collections, record data for each haul and then add the catch totals together. If you have trouble identifying organisms to the species level, list them at the most specific level of classification possible. TAKE PICTURES and submit those to us! Young of the year herring - alewife, blue-back herring, and American shad - look very similar to one another, as do very young sunfish. Group them together as herring or sunfish. Measure the largest individual of each species. It will not be possible to tell males from females for most of what you catch, but for a few - blue crabs for instance - it is possible and very useful to distinguish gender.

So that we can compare data from site to site please list:

Length of SEINE NET 50 ft    Length of SEINE PULL 7 yards    Total # of FISH PER HAUL  27

Total number of seines or catches you ran during your study period	1

If you site used traps, please note catches per trap:

If you use oyster cages please note this

If your site was visited by the fisheries team please note what fish came from this group

FISH  SPECIES: Use separate sheet for each seine OR note what was caught in each seine by noting seine #.

TIME:	12:30 pm      LENGTH OF NET: 		50 feet

METHOD: Seine: X.Traps:      Rod & Reel:      Electro-Shocking:      Trawl:      Dip net: 	    Other: (Explain)

Fish Species:				# of individuals:		Size of largest  (unit)

1. 	Striped Bass				5			5 inches

2. 	White Perch				2			6 inches

3. 	Atlantic Silverside 			20			3 inches

4. ____________________________	____________		___________  ____

5. ____________________________	____________		___________  ____

6. ____________________________	____________		___________  ____

7. ____________________________	____________		___________  ____

8. ____________________________	____________		___________  ____

MACROINVERTEBRATES: For crabs please note type (blue, mud, Asian etc.) and sex (M/F)

COLLECTION METHOD: Seine X. Kick Net      Trap      Eel Mop     Other (Explain) __________________

1.  	Blue Crab 			#    15 		Notes: all males

2.   	Green Crab 			#    2

3.  	Shore Shrimp 			#    30

4. ____________________________	________

5. ____________________________	________

To Compute Catch Per Unit Equivalent (CPUE) –  Let’s use a 50 ft. net for the example. Take length of net (50 ft.) X length of pull (example: 7 yards X 3 ft = 21 ft.). Convert to inches: 50 ft. X 21ft X 12 (for inches per foot) = total inches. Convert to meters: divide by 39.37 (for inches in a meter) = 320 meters.  Then divide your catch by 320 to get catch per meter seined.  This figure should be computed for each seine event.

If you pull the net in just to close a circle the formula is:  Net Length (ft.) ___X 12 = total inches / 39.37 (inches in a meter) =_____ Then divide your catch by this number for your CPUE.

Seine # 1    Time	12:30 pm   Catch Total  27   Length of Pull 21 ft 	        Computed CPUE: 	0.084

Hudson River Watershed Fish Fauna Check List

1 _____    lamprey, silver

2 _____    lamprey, American brook (n)

3 _____    lamprey, sea (n)

4 _____    shark (dusky shark?) (n)

5 _____    hammerhead, smooth (n)

6 _____    dogfish, smooth (n)

7 _____    dogfish, spiny (n)

8 _____    skate, little (n)

9 _____    skate, barndoor (n)

10 _____   stingray, bluntnose (n)

11 _____   ray, cownose

12 ____	sturgeon, shortnose (n)

13 ____	sturgeon, lake (n)

14 ____	sturgeon, Atlantic (n)

15 ____	gar, alligator

16 ____	gar, longnose

17 ____	bowfin

18 ____	ladyfish (n)

19 ____	tarpon

20 ____	bonefish (n)

21 ____	eel, American (n)

22 ____	worm eel, speckled (n)

23 ____	eel, conger (n)

24 ____	herring, blueback (n)

25 ____	shad, hickory (n)

26 ____	alewife (n)

27 ____	shad, American (n)

28 ____	menhaden, Atlantic (n)

29 ____	herring, Atlantic (n)

30 ____	shad, gizzard

31 ____	herring, round (n)

32 ____	thread herring, Atlantic (n)

33 ____	anchovy, striped (n)

34 ____	anchovy, bay (n)

35 ____	stoneroller, central

36 ____	goldfish

37 ____	dace, redside

38 ____	chub, lake (n)

39 ____	carp, grass

40 ____	shiner, satinfin (n)

41 ____	shiner, spotfin

42 ____	carp, common

carp, mirror (var.)

43 ____	Amur carp (var. koi)

44 ____	minnow, cutlip (n)

45 ____	minnow, brassy (n)

46 ____	minnow, eastern silvery (n)

47 ____	shiner, bridle (n)

48 ____	shiner, ironcolor (n)

49____		shiner, common (n)

50____		dace, pearl (n)

51 ____	chub, hornyhead

52 ____	shiner, golden (n)

53 ____	shiner, comely (n)

54 ____	shiner, emerald

55 ____	shiner, blackchin

56 ____	shiner, blacknose

57 ____	shiner, spottail (n)

58 ____	shiner, rosyface

59____		shiner, sand

60 ____	dace, northern redbelly (n)

61 ____	dace, finescale (n)

62 ____	minnow, bluntnose

63 ____	minnow, fathead

64 ____	dace, eastern blacknose (n)

65 ____	dace, longnose (n)

66 ____	chub, creek (n)

67 _____	fallfish (n)

68 ____	bitterling

69 ____	rudd

70 _____	sucker, longnose (n)

71 _____	sucker, white (n)

72 _____	sucker, summer (n)

73 _____	chubsucker, creek (n)

74 _____	hog sucker, northern (n)

_____	buffalo hybrid (black x smallmouth)

75 _____	redhorse, shorthead

76 _____	weatherfish, Oriental

77 _____	pirapitinga (red-bellied pacu)

78 _____	catfish, white (n)

79 _____	bullhead, yellow (n)

80 _____	bullhead, brown (n)

81 _____	catfish, channel

82 _____	stonecat

83 _____	madtom, tadpole (n)

84 _____	madtom, margined (n)

85 _____	madtom, brindled

86_____	pickerel, redfin (n)

87_____	pike, northern (n)

muskellunge, tiger (norlunge)

88 _____	pickerel, chain (n)

89 _____	mudminnow, central

90 _____	mudminnow, eastern (n)

91 ____	smelt, rainbow (n)

92 _____	herring, lake (cisco) (n)

93_____	whitefish, lake (n)

94_____	trout, rainbow

95 _____	kokanee (sockeye)

96 _____	salmon, chinook

97 _____	whitefish, round (n)

98 _____	salmon, Atlantic (n)

99 _____	trout, brown

100 _____	trout, brook (n)

_____	trout, tiger (brown x brook trout)

101 _____	trout, lake (n)

102 _____	lizardfish, inshore (n)

103 _____	trout-perch (n)

104 _____	rockling, fourbeard (n)

105 _____	cod, Atlantic (n)

106 _____	tomcod, Atlantic (n)

107 _____	pollock (n)

108 _____	hake, silver (whiting) (n)

109 _____	hake, red (ling) (n)

110 _____	hake, spotted (n)

111 _____	hake, white (n)

112 _____	cusk-eel, striped (n)

113 _____	toadfish, oyster (n)

114 _____	goosefish (anglerfish) (n)

115_____	needlefish, Atlantic (n)

116 _____	houndfish (n)

117 _____	minnow, sheepshead

118 _____	killifish, eastern banded (n)

119 _____	mummichog (n)

120 _____	killifish, striped (n)

121 _____	killifish, spotfin (n)

122 _____	mosquitofish, western

123 _____	silverside, brook

124 _____	silverside, rough (n)

125 _____	silverside, inland (n)

126 ____	silverside, Atlantic (n)

127 ____	stickleback, fourspine (n)

128 ____	stickleback, brook (n)

129 ____	stickleback, threespine (n)

130 ____	stickleback, ninespine (n)

131 ____	cornetfish, bluespotted (n)

132 ____	seahorse, lined (n)

133 ____	pipefish, northern (n)

134 ____	gurnard, flying (n)

135 ____	sea robin, northern (n)

136 ____	sea robin, striped (n)

137 ____	sculpin, slimy (n)

138 ____	sea raven (n)

139 ____	grubby (n)

140 ____	sculpin, longhorn (n)

141 ____	lumpfish (n)

142 ____	snailfish, inquiline (n)

143 ____	perch, white (n)

144 ____	bass, white

145 ____	bass, striped (n)

146 ____	sea bass, black (n)

147 ____	gag (grouper) (n)

148 ____	sunfish, mud (n)

149 ____   bass, rock

150 ____	sunfish, bluespotted (n)

151 ____	sunfish, banded (n)

152 ____	sunfish, redbreast (n)

153 ____	sunfish, green

154 ____	pumpkinseed (n)

155 ____	warmouth

156 ____	bluegill

157 ____	bass, smallmouth

158 ____	bass, largemouth

159 ____	crappie, white

160 ____	crappie, black

161 ____	darter, greenside

162_____ darter, rainbow

163 ____	darter, fantail

164 ____	darter, tessellated (n)

165 ____	perch, yellow (n)

166 ____	logperch, northern (n)

167 ____	darter, blackside

168 _____	darter, shield

169 ____	walleye

170 ____	bigeye, short (n)

171 ____	bluefish (n)

172 ____	cobia (n)

173 ____	sharksucker, live (n)

174 ____	sharksucker, whitefin (n)

175 ____	jack, crevalle (n)

176 ____	scad, round (n)

177 ____	moonfish, Atlantic (n)

178 ____	lookdown (n)

179____	rudderfish, banded (n)

180 ____	permit (n)

181 ____	schoolmaster (n)

182 ____	snapper, gray (mangrove) (n)

183 ____	mojarra, spotfin (n)

184 ____	pigfish (n)

185 ____	sheepshead (n)

186 ____	pinfish (n)

187 ____	scup (porgy) (n)

188 ____	drum, freshwater (sheepshead)

189 ____	perch, silver (n)

190 ____	weakfish (n)

191 ____	spot (Lafayette) (n)

192 ____	kingfish, northern (n)

193 ____	croaker, Atlantic (n)

194 ____	drum, black (n)

195 ____	butterflyfish, foureye (n)

196 ____	butterflyfish, spotfin (n)

197 ____	mullet, striped (n)

198 ____	mullet, white (n)

199 ____	sennet, northern (n)

200 ____	guaguanche (n)

201 ____	tautog (blackfish) (n)

202 ____	cunner (bergall, chogy) (n)

203 ____	gunnel, rock (n)

204 ____	sand lance, American (sand eel) (n)

205 ____	stargazer, northern (n)

206____	blenny, feather (n)

207 ____	blenny, freckled (n)

208 ____	skilletfish (n)

209 ____	sleeper, fat (n)

210 ____	goby, American freshwater

211 ____	goby, naked (n)

212 ____	goby, seaboard (n)

213 ____	goby, highfin (n)

214 ____	goby, round

215 ____	cutlassfish, Atlantic (n)

216 ____	mackerel, Atlantic (n)

217 ____	mackerel, Spanish (n)

218 ____	butterfish (n)

219 ____	snakehead, northern

220 ____	flounder, Gulf Stream (n)

221 ____	flounder, smallmouth (n)

222 ____	flounder, summer (fluke) (n)

223 ____	flounder, fourspot (n)

224 ____	flounder, yellowtail (n)

225 ____	flounder, winter (n)

226 ____	windowpane (n)

227 ____	hogchoker (n)

228 ____	tonguefish, northern (n)

229 ____	triggerfish, gray (n)

230 ____	filefish, orange (n)

231 ____	filefish, planehead (n)

232____   cowfish, scrawled (n)

233 ____	burrfish, striped (n)

234 ____	puffer, smooth (n)

235 ____	puffer, northern (n)

(n) = Native Species (180 – 0.77)

Taxonomy: Class-4   Order-27 Families-84

Genera-169   Species - 235

 Tom Lake NYSDEC Hudson River Estuary Naturalist trlake7@aol.com      August 15, 2021

ACTIVITY VII- JOURNALING & A HUDSON RIVER ALMANAC ENTRY

How do we learn about our natural environment?  We observe.

Through this activity we are focusing on developing skills of observation that play such an important role in science and Earth systems. Direct observation and careful description helps us compare species, habitats and different geographical regions. Through journaling we hope to observe, record and better understand some of the relationships that exist in the natural world.

JOURNALING:

What can we learn from journaling? If we record all our data as number on a data sheet, why do we need to do journaling as well? The development of observational skills is a key component in fieldwork, and is a basic building block in the successful use of the scientific method. Observations that are noted in the field are often good supplements to data, and can be used to further understand and examine collected results.

Let’s see what we can learn from journaling that would help us to better understand the numerical data recorded on our sheet:

- The water are calm on the Hudson, several Canadian geese swim at the water’s edge.

  - The calmness described suggests that there is little to no wind, the oxygen levels would not be affected by churning of water and weather related introductions. The observation of Canadian geese noted on the data recording sheet is a new bit of information.

- The rocks on which I sit are large and those at the edge with the water lapping around them are covered in green, slippery algae.

  - We learn that the water is up to the rocks- there was no water depth noted here or on the data recording sheet but this observation does tell us that during some part of the tidal cycle, there is some depth to the water. It also tells us the rocks are often wet since they are growing algae. This could suggest several things: 1. At low tide the water might still reach, or be close to these rocks, so they don’t get a chance to dry out much during a 24 hour period, 2. During high tide the water climbs high on these rocks and due to their positioning they don’t dry out well during low tide, 3. The rocks face away from the sun for a large portion of the day, 4. The rocks are stacked closely together so there is limited air movement between them so if they get wet they tend to stay wet allowing algae to grow.

HUDSON RIVER ALMANAC:

A compilation of journal entries from Hudson River fishermen, birders, and environmental observers. This journal is sent out to a list serve via email every weekly? You and your students are encouraged to contribute to this community journal!

Sample Hudson River Almanac entry (4-6 sentences):

September 29 - Dobbs Ferry - Our beach seine was filled with nearly 600 fish-snapper blues, white perch, a vast school of silverside, and several 4"-7" striped bass. Low flying monarchs passed in twos and threes, dipping within inches, brushing against us as they beat into strong southerlies. The students from Irvington were thrilled to be so close to so much loveliness. As they passed, the students called out the tally; they were moving past us at the rate of fifty an hour. Christopher Letts