Day in the Life of the Hudson River 10/13/22

Schodack Island Preserve, Schodack Landing

RIVER MILE: 134.5

Salt Front: ~ RM76

Columbia County Soil and Water CD  - Leanna O’Grady

Ichabod Crane High School – 14 students & 3 adults

Latitude - 42.499487°N & Longitude - 73.776702°W

Location: Schodack Island State Park, Schodack Landing, Rensselaer County, NY

Area: 1052 acres- park and boat launch area- grassy area with concrete walkway, pier, forested, grassy, parking lot and boat launch

Sampling Site: Boat launch, banks altered, covered in vegetation, riprap, concrete bulkhead, pipe entering the water

Surrounding Land Uses: 70% Forested, 15% Urban/Residential, 55% Industrial/Commercial

Shoreline: 5% Sandy, 15% Muddy, 80% Rocky

River Bottom: Rocky, Muddy

Water: NR

Plants in the Water: no plants

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature- |  | 11:00 AM |  | 16°C |  |  | 60.8°F |  |  |
| Wind Speed: |  | 10:58 AM |  | Anemometer 1-3 knots |  |  | Beaufort 1 | Direction S |  |
| Weather today |  | 10:45 AM |  | >75% cloud cover |  |  | Start & Stop |  |  |
| Weather the past 3 days: |  | No rain, no unusual wind or temps |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Water Height (cm) |  |  |
|  |  | 10:45 AM-12:30 PM |  | Falling the whole time |  |  | NR |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |  |
|  |  | 10:40 AM 10:43 AM 11:30 AM |  | 20.8 30.45 15.63 |  | NR | NR | Ebb |  |
| Water Temperature- |  | 10:55 AM |  | 15°C |  |  | 59°F |  |  |
| Turbidity- Secchi Disk (cm) |  | 10:45 AM |  | 133.3 cm |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity-LR Quantabs |  | 11:01 AM |  | 63 ppm |  |  | LR Qauntab 1.6 | 35 ppm Cl- |  |
| Standardized Salinity – LR Quantabs* note the unit conversions are matched to the batch of quantabs |  | 10:49 AM |  | 65 ppm |  |  | LR Quantab 1.6 | 36 ppm Cl- |  |
| DO – Lamotte Drop Count Kit |  | Time 10:40 AM |  | Temp 15°C |  |  | ppm DO 10.2 ppm | % saturation 100% |  |
| pH – pH pen |  | 10:50 AM 11:50 AM 11:53 AM |  | 6.3 *note fairly low 6.8 7.0 |  |  |  |  |  |
| Nitrates- NR |  |  |  |  |  |  |  |  |  |
| Phosphates- NR |  |  |  |  |  |  |  |  |  |
| Alkalinity- NR |  |  |  |  |  |  |  |  |  |
| Biological- |  |  |  |  |  |  |  |  |  |
| Fish – Seine – 2 pulls |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| 10:30 AM-12:00 PM |  | 10 1 15 23 1 |  | Blueback Herring Golden Shiner Spottail Shiner Banded Killifish Striped Bass |  |  |  |  |  |
| Fish Totals |  | TOTAL: 50 |  | Diversity: 5 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Macroinvertebrate Totals |  | TOTAL: 0 |  | Diversity: 0 |  |  |  |  |  |
| Sediment Core – NR |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| Boats- NR |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial | 12:40 PM |  | Edwin A Poling Being pushed by Kimberly Poling |  | White & teal |  | North | loaded |  |
| Recreational | 10:45AM |  |  |  | White |  | South |  |  |
|  | 11:15AM |  | DEC |  | Grey |  | North |  |  |
|  | 11:54AM |  | Riverkeeper |  | White |  | North |  |  |