Day in the Life of the Hudson River 10/13/22

Rensselear Boat Launch

RIVER MILE: 146

Salt Front: ~ RM76

Tech Valley High School– Ashley Phillips

75 students & 4 adults

Latitude 42.6575691°N & Longitude – 73.7331464°W

Location: Rensselear Boat Launch, Rensselear County, NY

Area: Rocky beach, dock; near hotel, train station, and train tracks

Sampling Site: sandy and rocky beach, parking lot

Surrounding Land Uses: 50% Urban/Residential, 50% Industrial/Commercial

Shoreline: Beach with sandy, small rocks, concrete boat launch

River Bottom: Muddy and Sandy

Water: Calm

Plants in the Water: NR

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature- |  | 11:00 AM 11:30 AM |  | 19°C 21°C |  |  | 67°F 69.8°F |  |  |
| Wind Speed: NR |  |  |  | Anemometer |  |  | Beaufort | Direction |  |
| Weather today |  | 11:00 AM |  | 26-51% cloud cover |  |  | No Rain |  |  |
| Weather the past 3 days: |  | Sunny |  |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Water Height (cm) |  |  |
|  |  | 11:30 AM 11:45 AM 12:00 PM |  | Falling Falling Falling |  |  | 18 cm 7 cm 2 cm |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |  |
| *all of these speeds seem high – perhaps missing a decimal? |  | 9:30 AM 10:00 AM 10:30 AM |  | 381 300 277 |  | NR | NR | NR |  |
| Water Temperature- |  | 9:38 AM 11:00 AM |  | 15°C 17°C |  |  | 59°F 62.6°F |  |  |
| Turbidity- Tabletop tube (JTUs) |  | 10:15 AM 10:45 AM 11:00 AM |  | 0 JTU 5 JTU 5 JTU |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity-Quantabs Meter/probe |  | 10:07 AM 10:07 AM 10:40 AM |  | 29 ppm Cl- 25 ppm (meter) 52.5 ppm (meter) |  |  | 1.4 quantab units | 52 ppm total salinity |  |
| Standardized Salinity – NR |  |  |  |  |  |  |  |  |  |
| DO – Meter/Probe |  | Time 11:30 AM 10:30 AM |  | Temp 15°C 17°C |  |  | ppm DO 9 ppm 8 ppm | % saturation 90% 80% |  |
| pH – Color Match Kit |  | 9:50 AM 10:16 AM 10:36 AM |  | 6.8 *pH seems low 6.4 6.4 |  |  |  |  |  |
| Nitrates- |  | 10:25 AM |  | 0.4 ppm |  |  |  |  |  |
| Phosphates- |  | 9:45 AM |  | 1.5 ppm average of two readings |  |  |  | *Average of two numbers |  |
| Alkalinity- titration |  | 9:50 AM |  | 90 ppm |  |  |  |  |  |
| Biological- Did Not Seine |  |  |  |  |  |  |  |  |  |
| Fish – Scott brought fish |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Fish Totals |  | TOTAL: |  | Diversity: |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Macroinvertebrate Totals |  | TOTAL: |  | Diversity: |  |  |  |  |  |
| Sediment Core – NR |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| Boats- NR |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial |  |  |  |  |  |  |  |  |  |
| Recreational |  |  |  |  |  |  |  |  |  |