Day in the Life of the Hudson River 10/13/22

Piermont Pier, Piermont

RIVER MILE: 25.4

Salt Front: ~ 76

Lamont Doherty Earth Observatory- Margie Turrin, Laurel Zaima, Marisa Annunziato

Tom Mullane & Pearl River HS, Shawn Kapusinsky & Spring Valley HS,

Alexis Dean & Clarkstown South HS, Liana Gertzer & Clarkstown North HS, and Brian Newburger & Tappan Zee HS

97 students & 14 adults

Latitude 41.043189 N & Longitude – 73.89595 W

Location: Piermont Pier, Piermont, Rockland, NY

Area: Human made fishing/shipping pier at the east end, mile long pier with paved roadway and vegetated strip along both sides

Surrounding Land Use: 80% Urban/Residential; 20% Forested

Sampling Site: Pier, roadway in center of whole pier, parking area at the end, plants and trees along side. Shoreline riprap, collected wood in area and piping entering river on the south of sampling site

Water Depth: Varied by location on the pier and by the tide

Plants in area: 15% sumac and other plants; 85% phragmites; no submerged aquatic plants in water

River Bottom: Sandy, rocky bottom with phragmites growing in or near the water

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |  |
| Air Temperature |  | 9:22 AM 10:00 AM 10:30 AM 12:26 PM |  | 20o C 19 o C 20 o C 20.4 o C |  |  |  |  |  |  |
| Wind Speed: |  | 9:22 AM 10:00 AM 10:30 AM 12:26 PM |  | Anemometer 6 mph 7 mph 11.6 mph 21.5 mph |  |  | Beaufort 2 3 3 NR | Direction SW SW SE NR |  |  |
| Weather today |  | 10:30 AM |  | > 75% cloud cover |  |  | Drizzle |  |  |  |
| Weather the past 3 days: |  | Sunny, cool, Drizzling, rainy |  |  |  |  |  |  |  |  |
| Tides- measure water level change |  | Time |  | Rising, Falling, Unchanged |  |  | Water Height (cm) |  |  |  |
| Group 1 Group 2 |  | 9:00 AM- 12:04 PM 9:01 AM-12:15 PM |  | Rising Rising |  |  | 340 cm 406 cm 373 cm 573 cm |  |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |  |  |
|  |  | 9:15 AM 9:50 AM 10:34 AM 10:50 AM 12:10 PM 12:20 PM 12:50 PM |  | 89.2 60.4 67.6 63.49 46.9 43.8 48.8 |  | 1.7 1.17 1.3 1.24 0.91 0.85 0.94 | North North North North North North North | Flood Flood Flood Flood Flood Flood Flood |  |  |
| Water Temperature- |  | 9:00 AM 10:00 AM 10:30 AM 12:00 PM 12:50 PM |  | 18°C GROUP 1 18.5° C 19° C 20° C 19° C |  |  | 9:15 AM 10:03 AM 10:30 AM NR 12:55 PM | 18°C GROUP 2 19° C 18° C NR 19° C |  |  |
| Turbidity- Long sight tube (120 cm) |  | 9:17 AM 9:37 AM 10:00 AM 10:33 AM 11:03 AM 12:08 PM 12:30 PM 12:55 PM 1:00 PM |  | 18.16 cm 18.6 cm 17 cm 16.4 cm 20 cm 27.7 cm 25.1 cm 18 cm 27.2 cm |  |  | Secchi Disk (time) 9:19 AM 9:45 AM 10:05 AM 11:10 AM 12:15 PM 12:30 PM 1:00 PM 1:15 PM |  | Secchi Disk (cm) 25.3 cm 32.0 cm 24.5 cm 31.7 cm 33 cm 42 cm 36 cm 46 cm |  |
| Chemical |  |  |  |  |  |  |  |  |  |  |
| Salinity- GROUP1 Refractometer |  | 9:00 AM 10:00 AM 10:30 AM 12:00 PM 12:55 PM |  | 11,000 ppm 8,000 ppm 11,000 ppm 11,000 ppm 10,000 ppm |  |  | 9:10 AM GROUP2 10:03 AM 10:30 AM 1:00 PM | 10,000 ppm 10,000 ppm 10,000 ppm 10,000 ppm |  |  |
| Standardized Salinity – Refractometer |  | 9:05 AM |  | 10,000 ppm |  |  |  |  |  |  |
| DO – Hach Drop Count Kit |  | Time |  | Temp |  |  | ppm DO | % saturation |  |  |
|  |  | 9:22 AM 9:30 AM 10:00 AM 10:03 AM 10:30 AM 10:30 AM 12:00 PM 12:00 PM 12:55 PM 1:00 PM |  | 18° C 18° C 19° C 19° C 18° C 19° C 20o C 20° C 19° C 19° C |  |  | 10 8 10 8 9 9 9 10 10 10 | NR 85% NR 85% NR 95% NR 110% 110% NR |  |  |
| pH – Color Match Kit |  | 9:20 AM 9:22 AM 10:00 AM 10:03 AM 10:30 AM 10:30 AM 12:00 PM 12:50 PM 12:55 PM |  | 8.5 8 8.5 8 7.5 7 8 8 7.5 |  |  |  |  |  |  |
| Nitrates- Lamotte Kit |  | 9:00 AM 9:20 AM 10:00 AM 10:03 AM 10:30 AM 10:30 AM 12:00 PM 12:00 PM 12:55 PM 1:00 PM |  | 0.8 ppm 0.8 ppm 0.8 ppm 0.4 ppm 0.6 ppm 0.5 ppm 0.6 ppm 0.4 ppm 0.4 ppm 0.4 ppm |  |  |  |  |  |  |
| Phosphates- Lamotte Kit |  | 9:00 AM 9:30 AM 10:00 AM 10:03 AM 10:30 AM 10:30 AM 12:00 PM 12:00 PM 1:00 PM |  | 0.64 ppm 0.5 ppm 0.2 ppm 0.5 ppm 0.22 ppm 0.55 ppm 0.28 ppm 0.32 ppm 0.62 ppm |  |  |  |  |  |  |
| Alkalinity- Lamotte Kit |  | 10:30 AM 12:00 PM 12:00 PM 12:55 PM 1:00 PM |  | 66 ppm 62 ppm 72 ppm 60 ppm 72 ppm |  |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |  |  |
| Fish – Seine |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |  |
| 9:20 AM- 1:15 PM |  | 253 5 4 1 |  | Atlantic Silverside Striped Bass Atlantic Menhaden Atlantic Herring |  |  |  |  |  |  |
| Fish Totals |  | TOTAL: 263 |  | Diversity: 4 |  |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |  |
|  |  | 8 (3M & 5F) ~1500 ~50 |  | Blue Crab Moon Jelly Ctenophore sp. |  |  |  |  |  |  |
| Macroinvertebrate Totals |  | TOTAL: 1558 |  | Diversity: 3 |  |  |  |  |  |  |
| Sediment Core |  |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |  |
| 9.5 cm 3.8 cm 13 cm 13 cm 4 cm 12 cm 13.6 cm 14.5 cm 6 cm | No No No No No Yes Yes Yes No |  | No No No No No No No No No |  | No Yes No Yes No Yes No Maybe No |  | Abundant: clay, mud, sand, gravel Common: pebbles Abundant: Mud Common: clay, gravel, pebbles Rare: sand Abundant: clay, mud Common: sand Rare: pebbles Abundant: sand Common: clay, mud, pebbles Rare: gravel Abundant: sand Common: gravel Rare: mud, pebbles Abundant: clay, mud, sand, gravel Rare: pebbles Abundant: clay, mud Common: sand Rare: gravel, pebbles Common: clay, mud Rare: sand, gravel, pebbles Abundant: sand, gravel Common: mud Rare: pebbles | Common: wood, shells Rare: leaves, macroinvertebrates, living vegetation, coal, brick, slag Rare: coal, living vegetation Common: living vegetation Abundant: leaves Common: wood, living vegetation Rare: coal Rare: brick, coal Common: wood Rare: brick, coal Common: leaves, living vegetation Common: living vegetation Rare: leaves, brick, coal Rare: leaves, wood, brick, living vegetation |  |  |
| Boats-NR |  |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |  |
| Commercial |  |  |  |  |  |  |  |  |  |  |
| Residential |  |  |  |  |  |  |  |  |  |  |