Day in the Life of the Hudson River 10/13/22

Nyack Memorial Park, Nyack, NY

RIVER MILE: 28

Salt Front: ~RM75

Nyack High School- Kirsten Kleinman

60 students & 15 adults

Latitude 41.08742 N & Longitude – 73.91576 W

Location: Nyack Memorial Park, Nyack

Area: Drainage pipe from Nyack Brook forming stream that leads into Hudson River

Surrounding Land Use: Forested, beach, park

Sampling Site: Pier, forested, beach, altered banks, grassy, covered in vegetation, RipRap, pipe entering the water

Shoreline: 30% Sandy; 70% Rocky

Water: Choppy

River Bottom: muddy, sandy, rocky

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |
| Air Temperature |  | 7:00 AM 11:00 AM |  | 15° C 20°C |  |  | 59°F 68°F |  |
| Wind Speed: NR |  | *Noted only that it was windy |  | Anemometer |  |  | Beaufort | Direction |
| Weather today |  | 9:00 -11:30 AM |  | > 75% cloud cover |  |  | Small Showers @ 10 AM |  |
| Weather the past 3 days: |  | Normal, sunny weather past 3 days |  |  |  |  |  |  |
| Tides- |  | Time |  | Rising, Falling, Unchanged |  |  | Water Height (cm) |  |
|  |  | 9:00 AM- 11:30 AM |  | Rising |  |  | 6 cm 38 cm | Low tide at 7 am High tide at 1 pm |
| Current- NR |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |
|  |  |  |  | *Noted pier and cove with stream flowing into it – not a good place to measure currents |  |  |  |  |
| Water Temperature- |  | 11:30 AM |  | 17.8°C |  |  |  |  |
| Turbidity- Secchi Disk (cm) |  | 11:30 AM |  | 103.5 cm |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |
| Salinity- |  | 11:30 AM |  | 10.6 ppt |  |  |  |  |
| Standardized Salinity – HR Quantabs |  | 11:50 AM |  | 8,536 ppm |  |  |  |  |
| DO – Chemets Kit |  | Time 11:30 AM |  | Temp 17.8°C |  |  | ppm DO 8 ppm | % saturation NR |
| pH – Meter |  | 11:30 AM |  | 6.7 |  |  |  |  |
| Nitrates- |  | 11:30 AM |  | 0 ppm |  |  |  |  |
| Phosphates- |  | 11:30 AM |  | 0.4 ppm |  |  |  |  |
| Alkalinity- NR |  |  |  |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |
| Fish – Seine |  | Number |  | Name |  |  | Length of longest | Length of smallest |
| 10:00-11:00 AM |  | 1 2 1 50 1 3 |  | Herring sp. Channel Catfish Banded Killifish Atlantic Silverside White Perch Striped Bass |  |  | 5 cm 35 cm 5 cm 10 cm 20 cm 12.7 cm | 25 cm 5 cm 5 cm |
| Fish Totals |  | TOTAL:78 |  | Diversity: 6 |  |  |  | 1 Snapping Turtle 30.4 cm |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |
|  |  | 2 20 |  | Blue Crab Ctenophore spp. |  |  | 5 cm 5 cm | 2 cm |
| Macroinvertebrate Totals |  | TOTAL: 22 |  | Diversity:2 |  |  |  |  |
| Sediment Core - |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |
| Boats-NR |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |

NYACK HS TRANSECT MEASUREMENTS – Brook to Hudson

9:00 AM LT

| Station # | Distance (M) | Depth (cm) | Salinity ppt | Temp °C | DO ppm | pH |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 6 | 0.6 | 18 | 10 | 7.6 |
| 2 | 10 | 10 | 0.6 | 17.6 | 10 | 8.1 |
| 3 | 20 | 21 | 0.6 | 17.6 | 10 | 7.4 |
| 4 | 30 | 54 | 4.8 | 17.3 | 8 | 7.2 |
| 5 | 40 | 115 | 5.6 | 17.8 | 8 | 6.8 |
| 6 | 50 |  | 4.9 | 17.4 | 7 | 7.3 |
| 7 | 60 |  | 2.3 | 17.4 | 6 | 6.9 |
| 8 | 70 | 63 | 7.3 | 17.2 | 8 | 7.0 |
| 9 | 80 | 43 | 2.9 | 17.3 | 7 | 7.4 |
| 10 | 90 | 28 | 4.6 | 17.7 | 8 | 6.4 |

|  | Nitrates | Phosphates |  | Turbidity cm visibility | Location |
| --- | --- | --- | --- | --- | --- |
| Station 1 | 2 ppm | 0 ppm |  | 93 cm | Butterfly garden |
| Station 10 | 2 ppm | 0.2 ppm |  | 83 cm | Shakey Dock |
|  |  |  |  | 103 cm | Far Pier |
| Salinity – not sure where? | Quantab reading | 2.8 |  | 392 ppm Cl- | 708 TS |

11:30 AM Rising Tide (HT 1PM)

| Station # | Distance (M) | Depth (cm) | Salinity ppt |  | Temp °C | DO ppm | pH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 38 cm | 2.6 |  | 18.3 | 9.0 | 7.3 |
| 2 | 10 | 52 cm | 2.7 |  | 18.0 | 8.0 | 7.5 |
| 3 | 20 | 87 cm | 4.4 |  | 17.6 | 8.0 | 7.4 |
| 4 | 30 | 100 cm | 6.8 |  | 17.6 | 8.0 | 7.4 |
| 5 | 40 | NR | 7.5 |  | 17.9 | 8.0 | 7.2 |
| 6 | 50 | NR | 6.7 |  | 17.6 | NR | 7.2 |
| 7 | 60 | NR | 8.0 |  | 17.4 | NR | 7.5 |
| 8 | 70 | 86 cm | 11.1 |  | 17.6 | NR | 6.5 |
| 9 | 80 | 52 cm | 10.6 |  | 18.3 | NR | 6.5 |
| 10 | 90 | 75 cm | 10.6 |  | 17.8 | NR | 6.7 |
|  | Nitrates | Phosphates |  | Turbidity cm visibility |  | Location |  |
| Station 1 | 2 ppm | 0.2 ppm |  | 103.5 cm |  | Butterfly garden |  |
| Station 10 | 0 ppm | 0.4 ppm |  | 94 cm |  | Shakey Dock |  |
|  |  |  |  | 93 cm |  | Far Pier |  |