Day in the Life of the Hudson River 10/13/22

George’s Island County Park, NY

RIVER MILE: 40

Salt Front: ~ RM76

Fox Meadow Elementary School– Alethea Lynch

100 students & 30 adults

Latitude 41.24936 N & Longitude -73.936474 W

Location: George’s Island County Park, Westchester County, NY

Area: waterfront park, banks altered, bulkhead with wooden timbers, boat launch

Surrounding Land Use: Primarily, park picnic area and boat launch; forested area; parking lot

Sampling Site: Beach, riprap, and concrete bulkhead

Shoreline: sandy rocky with brick pieces

Plants in area: grass covers > than ½ of site – low cut, deciduous trees cover ¼ of site include sugar maples, black locust, willows, hophorn beam, Common reed (Phragmites) covers

River Bottom: muddy

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |
| Air Temperature- |  | 11:15 AM 11:36 AM 11:45 AM 12:30 PM |  | 20°C 20 19 18 |  |  | 68°F 68 67 60 |  |
| Wind Speed: |  | 11:15 AM 11:43 AM |  | Anemometer 1.4 mph 2 mph |  |  | Beaufort 3 3 | Direction S S |
| Weather today |  | 11:15 AM |  | 51-75% cloud cover |  |  | Rain after 11:45 AM |  |
| Weather the past 3 days: |  | Warm, clear |  |  |  |  |  |  |
| Tides- NR |  | Time |  | Rising, Falling, Unchanged |  |  | Water Height (cm) |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |
|  |  | 11:15 AM 11:30 AM 11:45 AM 12:30 PM |  | NR |  | NR | NR | Flood Flood Flood Flood |
| Water Temperature- |  | 10:40 AM 11:15 AM 11:45 AM |  | 17°C 15°C 19°C |  |  | 60°F 60°F 65°F |  |
| Turbidity- Secchi Disk (cm) |  | 11:05 AM |  | 36 cm |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |
| Salinity-Hydrometer* |  | 10:13 AM 11:11 AM 11:45 AM 12:26 PM 12:38 PM |  | 3 ppt 6 ppt 10 ppt 11 ppt 12 ppt |  |  | *expect salt built up in the hydrometer as it go higher throughout the day |  |
| Standardized Salinity – NR |  |  |  |  |  |  |  |  |
| DO – Drop Count Kit |  | Time 10:30 AM 11:15 AM 12:36 PM |  | Temp 16°C 15°C 17°C |  |  | ppm DO 5 ppm 4 ppm 4 ppm | % saturation NR NR |
| pH – Test Strips |  | 10:07 AM 10:57 AM 11:15 AM |  | 6 7 7 |  |  |  |  |
| Nitrates- NR |  |  |  |  |  |  |  |  |
| Phosphates- NR |  |  |  |  |  |  |  |  |
| Alkalinity- NR |  |  |  |  |  |  |  |  |
| Biological- |  |  |  |  |  |  |  |  |
| Fish – Seine |  | Number |  | Name |  |  | Length of longest | Length of smallest |
| 11:30 AM-12:30 PM |  | 2 19 3 4 |  | American Eel Atlantic Silverside White Perch Striped Bass |  |  |  |  |
| Fish Totals |  | TOTAL: 28 |  | Diversity: 4 |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |
|  |  | 5 1 1 1 |  | Green Crab* Lady Crab* Moon Jellies Palaemonetes |  |  |  |  |
| Macroinvertebrate Totals |  | TOTAL: 8 |  | Diversity:4 |  |  |  |  |
| Sediment Core – NR |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |
| Boats- NR |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |
| Commercial |  |  |  |  |  |  |  |  |
| Recreational |  |  |  |  |  |  |  |  |