Day in the Life of the Hudson River 10/13/22

Pier I, NYC

RIVER MILE: 6.8

Salt Front: ~ RM 76

Speyer Legacy School (7th grade)– Kimberely Schwab

36 students & 4adults

Latitude 40.7099° N & Longitude – 73.9845°W

Location: Pier I, NYC

Area: Pier was built on a dormer railroad pier jutting out at an angle from Riverside Park South.

Surrounding Land Use: Urban/Residential

Sampling Site: Pier

Shoreline: Sandy, rocky

Water: Choppy

Plants in area: No plants in the water

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature- |  | 10:00 AM 10:30 AM 11:00 AM |  | 19.44°C 20°C 20°C |  |  | 67°F 68°F 68°F |  |  |
| Wind Speed: |  | 10:00 AM |  | Anemometer NR |  |  | Beaufort 2 | Direction NR |  |
| Weather today |  | 11:00 AM |  | >75% cloud cover |  |  | Intermittent light rain |  |  |
| Weather the past 3 days: |  | Sunny, beautiful, rainless |  |  |  |  |  |  |  |
| Tides- NR |  | Time |  | Rising, Falling, Unchanged |  |  | Water Height (cm) |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |  |
|  |  | 10:00 AM 10:30 AM 11:00 AM 11:30 AM |  | 37.6 58 69.5 26.9 |  | 0.73 1.12 1.35 0.52 | NR | Flood Flood Flood Flood |  |
| Water Temperature- |  | 10:00 AM 10:15 AM 10:30 AM |  | 17°C 19°C 18°C |  |  |  |  |  |
| Turbidity- Long Sight Tube (120 cm) |  | 10:00 AM 10:30 AM 11:00 AM |  | 53.6 cm 30.26 cm 34.1 cm |  |  |  |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity-Hydrometer |  | 10:00 AM 10:30 AM 11:00 AM |  | 22 ppt 18 ppt 21 ppt |  |  |  |  |  |
| Standardized Salinity – |  |  |  |  |  |  |  |  |  |
| DO – Hach Drop Kit |  | Time 10:00 AM 10:30 AM 11:00 AM |  | Temp NR |  |  | ppm DO 9 ppm 6 ppm 7 ppm | % saturation NR |  |
| pH – |  | 10:00 AM 10:30 AM 11:00 AM |  | 7.5 7.5 7.5 |  |  |  |  |  |
| Nitrates- NR |  |  |  |  |  |  |  |  |  |
| Phosphates- NR |  |  |  |  |  |  |  |  |  |
| Alkalinity- NR |  |  |  |  |  |  |  |  |  |
| Biological- |  |  |  |  |  |  |  |  |  |
| Fish – Fish Traps |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
|  |  | 3 |  | Cunner |  |  | 9 cm | 7.6 cm |  |
| Fish Totals |  | TOTAL: 3 |  | Diversity: 1 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
|  |  | 9 (3F, 6M) |  | Blue Crab |  |  | 14 cm | 12.7 cm |  |
| Macroinvertebrate Totals |  | TOTAL: 9 |  | Diversity: 1 |  |  |  |  |  |
| Sediment Core - NA |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| Boats-NR |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |