Day in the Life of the Hudson River 10/13/22

People’s Waterfront, Newburgh, NY

RIVER MILE: 60.5

Salt Front: ~ RM76

Scenic Hudson– Katie Phipps

& NFA PTech - Joyce D'Imperio

55 students & 6 adults

Latitude 41.4934 N  Longitude – 74.00741 W

Location: Newburgh Waterfront- People’s Waterfront and Boat Launch, Newburgh, Orange County, NY

Area: Pier, dock with bulkhead, riprap, and wood bulkhead d

Surrounding Land Use: 75% Industrial/Commercial; 25% Park

Sampling Site: Shore with bulkhead, banks altered with riprap, debris in area

Shoreline: rocky

River Bottom: Muddy, rocky

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |
| Air Temperature |  | 10:25 AM |  | 20° C |  |  | 68°F |  |
| Wind Speed: |  | 10:20 AM |  | Anemometer 48 mph |  |  | Beaufort 9 | Direction NR |
| Weather today |  | 10:25 AM |  | > 75% cloud cover |  |  | Drizzle |  |
| Weather the past 3 days: |  | Sunny fall day, high 60s, rain on DITL |  |  |  |  |  |  |
| Tides- NR |  | Time |  | Rising, Falling, Unchanged |  |  | Water Height (cm) |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |
|  |  | 10:45 AM 11:15 AM |  | 1.03 0.81 |  | 0.2 0.2 | North North | Flood Flood |
| Water Temperature- |  | 11:18 AM |  | 17°C |  |  | 62°F |  |
| Turbidity- Long Sight Tube (120 cm) |  | 11:20 AM |  | 43 cm |  |  | Long sight tube 35 cm Secchi 40 cm |  |
| Chemical |  |  |  |  |  |  |  |  |
| Salinity- LR Quantabs |  | 10:11 AM 10:48 AM 11:18 AM |  | 489 ppm Cl- 423 ppm Cl- 342 ppm Cl- |  |  | 883 ppm total salt 764 ppm 617 ppm |  |
| Standardized Salinity – HR Quantab - NR |  |  |  |  |  |  |  |  |
| DO – Hach Drop Count Kit |  | Time 10:11 AM |  | Temp 17°C |  |  | ppm DO 10 ppm | % saturation 80% |
| pH – Color Match Kit |  | 10:11 AM 10:48 AM 11:18 AM |  | 8.2/7.4 7.6 7.4 |  |  |  | Avg 7.8 |
| Nitrates- NR |  |  |  |  |  |  |  |  |
| Phosphates- NR |  |  |  |  |  |  |  |  |
| Alkalinity- NR |  |  |  |  |  |  |  |  |
| Biological |  |  |  |  |  |  |  |  |
| Fish – Seine |  | Number |  | Name |  |  | Length of longest | Length of smallest |
| 11:00 AM |  | 184 12 22 2 1 |  | Herring sp. Spottail Shiner Atlantic Silverside White Perch Striped Bass |  |  | 12 cm 15 cm |  |
| Fish Totals |  | TOTAL: 221 |  | Diversity: 5 |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |
| Macroinvertebrate Totals |  | TOTAL: 0 |  | Diversity: 0 |  |  |  |  |
| Sediment Core - |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |
| Boats- |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |