Day in the Life of the Hudson River 10/13/22

Wayras Park Boat Launch, Poughkeepsie, NY

RIVER MILE: 75.5

Salt Front: ~ RM 76

Oakwood Friends– Amelia Goldstein

35 students & 4 adults

N 417056’ – W -739403’

(Latitude 41° 42’32.80’’N & Longitude – 73°56’25.19’’W)

Location: Wayras Park Boat Launch, Poughkeepsie, Dutchess County, NY

Area: public park, pier, grassy, parking lot. Park used for fishing, small boat launch, another pier for small boat launching. Near the walkway over the Hudson in between two bridges in the Mid-Hudson

Sampling Site: Park Pier, parking lot

Surrounding Land Uses: 100% Urban/Residential

Shoreline: 100% Rocky

River Bottom: NR

Water: NR

Plants in the Water: NR

*NR = Not read/recorded

| ITEM |  | Time |  | Reading |  |  |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |  |  |  |  |
| Air Temperature- |  | 11:10 AM |  | 19.2°C |  |  | 66°F |  |  |
| Wind Speed: |  | 11:25 AM |  | Anemometer 3.2 m/s |  |  | Beaufort NR | Direction N |  |
| Weather today |  | 11:30 AM |  | >75% cloud cover |  |  | Steady Rain |  |  |
| Weather the past 3 days: |  | Sunny, pleasant |  |  |  |  |  |  |  |
| Tides- Measured from dock to the water’s surface |  | Time |  | Rising, Falling, Unchanged |  |  | Water Height (cm) |  |  |
|  |  | 11:15 AM-12:22 PM |  | Rising |  |  | 74 cm 72.5 cm |  |  |
| Current- |  | Time |  | Cm/sec |  | Knots | North/South | Ebb/Flood/Still |  |
|  |  | 11:20 AM 1:48 PM |  | 19.76 12.6 |  | 0.29 NR | NR NR | Flood Flood |  |
| Water Temperature- |  | 11:00 AM |  | 25°C |  |  | 77°F |  |  |
| Turbidity- Long Sight Tube (120 cm) |  | 10:51 AM 11:17 AM 11:21 AM 11:25 AM |  | 33 cm 110 cm 106 cm 77 cm |  |  | 10:51 AM secchi disk – 57 cm 11:17 AM Secchi disk – 110 cm |  |  |
| Chemical |  |  |  |  |  |  |  |  |  |
| Salinity-NR |  |  |  |  |  |  |  |  |  |
| Standardized Salinity– LR Quantabs |  | NR |  | 96 ppm |  |  |  |  |  |
| DO – NR |  | Time |  | Temp |  |  | ppm DO | % saturation |  |
| pH – NR |  |  |  |  |  |  |  |  |  |
| Nitrates- NR |  |  |  |  |  |  |  |  |  |
| Phosphates- NR |  |  |  |  |  |  |  |  |  |
| Alkalinity- NR |  |  |  |  |  |  |  |  |  |
| Biological- |  |  |  |  |  |  |  |  |  |
| Fish – Seine |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Fish Totals |  | TOTAL: 0 |  | Diversity: 0 |  |  |  |  |  |
| Macroinvertebrates |  | Number |  | Name |  |  | Length of longest | Length of smallest |  |
| Macroinvertebrate Totals |  | TOTAL: 0 |  | Diversity:0 |  |  |  |  |  |
| Sediment Core – |  |  |  |  |  |  |  |  |  |
| Length | H2S smell |  | Oxid Top |  | Oil |  | Grain Size | Other Features |  |
| NR | Yes |  | No |  | No |  | Abundant: sand, gravel, pebbles Common: mud Rare: clay | Rare: brick |  |
| Boats- NR |  |  |  |  |  |  |  |  |  |
|  | Time |  | Name |  | Color |  | North/Southbound | Light/Loaded |  |
| Commercial |  |  |  |  |  |  |  |  |  |
| Recreational |  |  |  |  |  |  |  |  |  |