Day in the Life of the Hudson River 10/05/2023

Hudson River Park Pier 84, Manhattan, NYC

RIVER MILE: 5.6

Salt Front: ~ RM 52

Hudson River Park’s River Project-Toland Kister - 2 adults

Latitude 40.764608°N & Longitude –74.000069°W

Location: Hudson River Park Pier 84, Manhattan, NYC

Surrounding Land Use: Park with a mix of grass lawn and concrete surfaces (bulkhead), 60% Urban/Residential, 40% Industrial/Commercial

Sampling Site: Recreational Pier, Concrete Bulkhead

Shoreline: NR

Water: Choppy

River Bottom: Muddy, 400-600 cm deep

Plants in the water: No Plants in the water

*NR = Not read/recorded

| ITEM | Time | Reading |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical |  |  |  |  |  |
| Air Temperature | 1:30 PM | 23o C |  | 73 o F |  |
| Wind Speed: | 1:35 PM | Anemometer 10 knots |  | Beaufort 3 | Direction Northwest |
| Weather today | 1:30 PM | <25% Cloud Cover |  | No Rain |  |
| Weather the past 3 days: | Clear |  |  |  |  |
| Tides- NR | Time | Rising, Falling, Unchanged |  | Height (cm) |  |
| Current- NR | Time | Cm/sec | Knots | North/South | Ebb/Flood/Slack |
| Water Temperature- | 1:40 PM | 22°C |  | 71.6°F |  |
| Turbidity- Long Sight Tube (120 cm) | 1:45 PM | 60 cm |  |  |  |
| Chemical |  |  |  |  |  |

| Partner Salinity- Hydrometer | 1:40 PM | 12,000 ppm |  |  | 12 ppt |  |
| --- | --- | --- | --- | --- | --- | --- |
| DO – Lamotte Drop Count Kit | Time | Temp |  |  | ppm DO | % saturation |
|  | 2:00 PM | 22°C |  |  | 5.9 ppm | 65% |
| pH – NR |  |  |  |  |  |  |
| Biological- Did not fish |  |  |  |  |  |  |
| Fish | Number | Name |  |  | Length of longest | Notes |
| Fish Totals | TOTAL: | Diversity: |  |  |  |  |
| Macroinvertebrates | Number | Name |  |  | Length of longest |  |
| Macroinvertebrate Totals | TOTAL: | Diversity: |  |  |  |  |
| Sediment Core -Not a site that can core | Time | Oxid Top | H2S Smell | Oil | Grain Size | Other Features |

| Boats-NR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Commercial/Recreational | Time | Name | Color | Southbound/Northbound | Light/Loaded |